Congratulations on your selection of our company products!

This manual provides you with the general information you need start using your products. Consult your dealer or our company if you still have question or doubt after read this manual.

Be sure to keep all user of documentation handy for future reference.

Technical parameter

Voltage: AC220V/50 Hz (110V available)

Power: 70W

Control mode: DMX

DMX channel: 2

Working pressure: 5 MPa

Effect height: 8-10 M

Size: 30*23*18 (cm)

Weight: 9 KG

It can spray CO2 fog in swing horizontal 180 degrees.

Installation instructions

- Put the machine on the corresponding position;

- Connect the pipeline with quick connector in the machine (push back the sleeve, point the pipeline into connector of the machine, then release the sleeve);

- Connect the pipeline with gas cylinder;

- Lying down the gas cylinder to let the end of it higher a little;

- Open the value of the gas cylinder;

- Connect the machines with controller through control cables;

- When uninstall, close the valve of CO2 gas cylinder first, press button of controller to make machines spray CO2 again to let the residual gas out, then turn off the power, take down the pipeline.

Operation instructions

Set DMX address of each unit in advance;

Turn on the power of DMX controller, press button of DMX controller, then the machine works.

Notice:

To spray CO2 fog continuously no more than 2 seconds.

Then interval time of two shots ins not less than 5 seconds.

Notice

- It is must to use food-class CO2 liquid gas to prevent the damage of impurity and water to the pipeline and valve of machine.

- Fill in CO2 gas under international standard: 0.6kg/L.

- If using regular gas cylinder, please lie down it when working.

- To spray CO2 fog continuously no more than 2 seconds.

- The interval time of two shots is not less than 5 seconds.

- Make sure every connection of pipeline is reliable!

- Clean the machine after using.

LED CO2 SWING

Manual

Please read this manual carefully before using