一、显示面板及按键定义

MENU        UP       DOWN      ENTER

Menu key: Select function

Up key: parameter addition

Down key: decrease the parameter

Confirm key: confirm and save

二、Menu functions

After power on, press the menu key to display the menu menu in turn; The up or down key is used to modify the function parameters, and the confirmation key is used to save the current functions and parameters (power down memory is available after saving).

Menu Menu：

| A001 |  | A512 | Set the address code, modify the address code (A001~A512) upward or downward, and press OK to save. |
| --- | --- | --- | --- |
| CH16 |  | CH16 | Switch the five channels CH03, CH16, CH22, CH28 and ch40 up or down, and press OK to save. Default CH16 |
| A007 |  | N63 | Auxiliary light effect mode, confirm to save |
| S000 |  | S000 | Modify the running speed (S000~S255) of secondary lighting effect up or down, and press OK to save. |
| E007 |  | E21 | In the middle main light mode, confirm to save. |
| S000 |  | S255 | Modify the running speed of the intermediate main beam (S000~S255) up or down, and press OK to save. |
| Soud |  | Soud | Voice mode. |
| R255 |  | R000 | R The red lamp bead dims. Modify the brightness of the red lamp bead (R000~R255) upward or downward, and press OK to save. |
| G255 |  | G000 | G Adjust the light of green lamp bead, modify the brightness of green lamp bead (G000~G255) upward or downward, and press OK to save. |
| B255 |  | B000 | B Adjust the light of the blue lamp bead, modify the brightness of the blue lamp bead (B000~B255) upward or downward, and press OK to save. |
| W255 |  | W000 | W monochrome lamp brightness, modify the monochrome lamp brightness up or down (W000~W255), and press OK to save. |

三、Master slave control

Two or more identical lamps are connected with DMX three core signal lines. All lamps are set with any address code from A001 to A512. Any lamp is set as the host, and other lamps are slave. All slave displays do not flicker; When the master is used to adjust the effect of gradual change, pulse change, jump change, voice control and self walking, all slave machines are synchronized with the effect of gradual change, pulse change, jump change, voice control and self walking.

Special attention: 1. Only one host can be set for a group of lamps. If there are multiple hosts, all lamps will flash randomly and not synchronize.

2. All lamps can only work when DMX512 console is turned off.

四、Factory Settings

When any address code is A001~A512, press the menu key for 3 seconds to enter the factory setting. Factory setting mainly includes the functions of output power of each circuit of lamps, fan setting mode, setting temperature protection point, and sending parameters. Press the menu key for 3 seconds to exit any mode set in the factory.

Factory Setting Table：

| R255 |  | R020 | R Red lamp bead current setting, modify the hexagon red lamp bead current (R020-R255) upward or downward, and press OK to save. |
| --- | --- | --- | --- |
| G255 |  | G020 | G Green lamp bead current setting, modify the hexagon green lamp bead current (G020-G255) upward or downward, and press OK to save. |
| B255 |  | B020 | B Set the blue lamp bead current. Modify the hexagonal blue lamp bead current (B020-B255) up or down, and press OK to save it. |
| W255 |  | W020 | W monochrome lamp current setting, modify monochrome lamp current setting up or down (W020~W255), and press OK to save. |
| FAN1 |  | FAN0 | Fan setting: start the fan when FAN0 light is on, start the fan when FAN1 reaches the set temperature protection point, and press OK to save. |
| T040 |  | T-105 | Set the temperature protection point, modify the parameters up or down (40 ℃~75 ℃), and press the OK key to save. |
| Send |  | Send | Send the factory setting parameters of the machine up or down to the lamps connected in parallel with all other three core signal lines; Confirm sending parameters and press the menu key for 3 seconds |

五、DMX512Console

After power on, the address codes of all lamps are set, and all lamps are connected to DMX512 console in parallel with three core signal lines. The address codes will stop flashing, indicating that DMX512 console signals have been sent to the lamps. DMX512 console is used to control relevant functions according to the instructions of each channel.

CH04Channel Description：

| passageway | Channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | Linear dimming of red lamp beads. |
| 2 | 000-255 | Green lamp bead linear dimming. |
| 3 | 000-255 | Linear dimming of blue lamp beads. |
| 4 | 000-255 | Linear dimming of white lamp beads. |

CH16Channel description: factory default channel

| passageway | Channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | General dimming |
| 2 | 000-255 | Red green blue total dimming |
| 3 | 000-255 | Linear dimming of red lamp beads. |
| 4 | 000-255 | Green lamp bead linear dimming. |
| 5 | 000-255 | Linear dimming of blue lamp beads. |
| 6 | 000-255 | Red green blue stroboscopic (0-9 does not work, 10-255 speeds up) |
| 7 | 000-255 | Red, green and blue mode. Effect (4 values for one effect) |
| 8 | 000-255 | Red, green and blue speed |
| 9 | 000-255 | Red green blue background color |
| 10 | 000-255 | Red green blue background dimming |
| 11 | 000-255 | White lamp bead dimming |
| 12 | 000-255 | White lamp beads flicker |
| 13 | 000-255 | White lamp bead mode, effect (0-5 no effect) (12 values, 1 effect) |
| 14 | 000-255 | White lamp bead mold speed |
| 15 | 000-255 | Red green blue white combination mode, effect |
| 16 | 000-255 | Red green blue white combination (speed) |

CH21Channel Description：

| passageway | Channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | General dimming |
| 2 | 000-255 | Red green blue total dimming |
| 3 | 000-255 | Linear dimming of red lamp beads. |
| 4 | 000-255 | Green lamp bead linear dimming. |
| 5 | 000-255 | Linear dimming of blue lamp beads. |
| 6 | 000-255 | Red green blue strobe |
| 7 | 000-255 | Red green blue mode. Effect (4 values and 1 effect) |
| 8 | 000-255 | Red, green and blue speed |
| 9 | 000-255 | Red, green and blue - background color |
| 10 | 000-255 | Red, green and blue - background dimming |
| 11 | 000-255 | Segment 01 White Beads Linear Dimming. |
| 12 | 000-255 | Section 02 White Beads Linear Dimming. |
| 13 | 000-255 | Section 03 White Beads Linear Dimming. |
| 14 | 000-255 | Segment 04 White Beads Linear Dimming. |
| 15 | 000-255 | Segment 05 White Beads Linear Dimming. |
| 16 | 000-255 | Section 06 White Beads Linear Dimming. |
| 17 | 000-255 | White strobe |
| 18 | 000-255 | White light mode. Effect (0-5 useless. 12 values and 1 effect) |
| 19 | 000-255 | White light velocity |
| 20 | 000-255 | Red green blue white combination mode. effect |
| 21 | 000-255 | Red green blue white speed |

CH24Channel Description：

| 1 | 000-255 | Section 01: Linear dimming of red lamp beads. |
| --- | --- | --- |
| 2 | 000-255 | Section 01: Linear dimming of green lamp beads. |
| 3 | 000-255 | Section 01: Linear dimming of blue lamp beads. |
| 4 | 000-255 | Section 02: Linear dimming of red lamp beads. |
| 5 | 000-255 | Section 02 Linear Dimming of Green Beads. |
| 6 | 000-255 | Section 02: Linear dimming of blue lamp beads. |
| 7 | 000-255 | Section 03 Linear Dimming of Red Beads. |
| 8 | 000-255 | Section 03 Linear Dimming of Green Beads. |
| 9 | 000-255 | Section 03 Linear Dimming of Blue Beads. |
| 10 | 000-255 | Section 04: Linear dimming of red lamp beads. |
| 11 | 000-255 | Section 04 Linear dimming of green lamp beads. |
| 12 | 000-255 | Section 04 Linear dimming of blue lamp beads. |
| 13 | 000-255 | Section 05 Linear dimming of red lamp beads. |
| 14 | 000-255 | Segment 05 Linear Dimming of Green Beads. |
| 15 | 000-255 | Segment 05 Blue Beads Linear Dimming. |
| 16 | 000-255 | Section 06 Linear dimming of red lamp beads. |
| 17 | 000-255 | Section 06 Linear Dimming of Green Beads. |
| 18 | 000-255 | Section 06 Linear dimming of blue lamp beads. |
| 19 | 000-255 | Segment 01 White Beads Linear Dimming. |
| 20 | 000-255 | Section 02 White Beads Linear Dimming. |
| 21 | 000-255 | Section 03 White Beads Linear Dimming. |
| 22 | 000-255 | Segment 04 White Beads Linear Dimming. |
| 23 | 000-255 | Segment 05 White Beads Linear Dimming. |
| 24 | 000-255 | Section 06 White Beads Linear Dimming. |

CH36Channel Description：

| 1 | 000-255 | General dimming |
| --- | --- | --- |
| 2 | 000-255 | Red, green and blue - total dimming |
| 3 | 000-255 | Section 01: Linear dimming of red lamp beads. |
| 4 | 000-255 | Section 01: Linear dimming of green lamp beads. |
| 5 | 000-255 | Section 01: Linear dimming of blue lamp beads. |
| 6 | 000-255 | Section 02: Linear dimming of red lamp beads. |
| 7 | 000-255 | Section 02 Linear Dimming of Green Beads. |
| 8 | 000-255 | Section 02: Linear dimming of blue lamp beads. |
| 9 | 000-255 | Section 03 Linear Dimming of Red Beads. |
| 10 | 000-255 | Section 03 Linear Dimming of Green Beads. |
| 11 | 000-255 | Section 03 Linear Dimming of Blue Beads. |
| 12 | 000-255 | Section 04: Linear dimming of red lamp beads. |
| 13 | 000-255 | Section 04 Linear dimming of green lamp beads. |
| 14 | 000-255 | Section 04 Linear dimming of blue lamp beads. |
| 15 | 000-255 | Section 05 Linear dimming of red lamp beads. |
| 16 | 000-255 | Segment 05 Linear Dimming of Green Beads. |
| 17 | 000-255 | Segment 05 Blue Beads Linear Dimming. |
| 18 | 000-255 | Section 06 Linear dimming of red lamp beads. |
| 19 | 000-255 | Section 06 Linear Dimming of Green Beads. |
| 20 | 000-255 | Section 06 Linear dimming of blue lamp beads. |
| 21 | 000-255 | Red green blue strobe |
| 22 | 000-255 | Red, green and blue - mode (4 values and 1 mode) |
| 23 | 000-255 | Red, Green and Blue - Speed |
| 24 | 000-255 | Background color |
| 25 | 000-255 | Background tone light |
| 26 | 000-255 | Segment 01 White Beads Linear Dimming. |
| 27 | 000-255 | Section 02 White Beads Linear Dimming. |
| 28 | 000-255 | Section 03 White Beads Linear Dimming. |
| 29 | 000-255 | Segment 04 White Beads Linear Dimming. |
| 30 | 000-255 | Segment 05 White Beads Linear Dimming. |
| 31 | 000-255 | Section 06 White Beads Linear Dimming. |
| 32 | 000-255 | White light strobe |
| 33 | 000-255 | White light - mode, effect (0-5 useless. 12 values, 1 effect) |
| 34 | 000-255 | White light - speed |
| 35 | 000-255 | Red green blue white combination mode. effect |
| 36 | 000-255 | Red green blue white speed |

六、Mode effect

| Channel value | Mode code | effect |
| --- | --- | --- |
| 0-7 | 0 | No effect |
| 8-15 | 1 | Effect 1 |
| 16-23 | 2 | Effect 2 |
| 24-31 | 3 | Effect 3 |
| 32-39 | 4 | Effect 4 |
| … | … | … … |
| 232-239 | 29 | Effect 29 |
| 240-247 | 30 | Mode code 1-29 cycles. |
| 248-255 | 31 | Voice control |

七、Technical parameters:

Voltage: AC100~220V 50/60HZ

Power: 144 * 0.3W+6 * 50W (actual power: 200W)

Beads: 144 5050 tri color LED beads+6 warm white COB beads

Control mode: DMX512, self-propelled, master-slave, voice control, with RDM function.

Channels: CH04, CH16, CH22, CH28, CH40

Dimming: 32bit 0~100% linear dimming

Features: dyeing+flash+auxiliary light

Operating temperature: - 30 ℃~50 ℃

Stroboscopic frequency: 1~30HZ

Appearance: metal, black

Connection mode: DMX512 input/output/power input/output.

IP grade: IP20

Size: Weight: