SUPERSTAR STROBE

USER MANUAL

TECHNICAL PARAMETERS:

Voltage: AC 100-240V,50/60Hz

Main light:12*60W(RGBW)full-color and high brightness dyeing effect(15degrees),adjustable for horse racing and point control

Strobe: 600W pure white light strobe effect (8-segment flowing)

Auxiliary light pixel bar:

3W* 32 (RGBW) full-color pixel bar effect (8 segments);

3W* 32 amber (8 segments)

Channel:10CH/46CH/119CH

Control:DMX512,AUTO,SOUND,MASTER-SLAVE

Menu functions:

ADDR		A001---A512 	Address code

CHND		10CH, 46CH, 119CH  Channel selection

SLND			MAST, SLAV

SHND		SOUD, AUTO(SP0---SP9)

SENS			Sound control sensitivity (0 off, 100 most sensitive)

LED			ON, OFF		Backlight switch

DISP			YES, NO 	Reverse display

TEST			Automatic testing

TEMP			0----100°	Temperature display

DMX 10 Channel function:

1. Total dimming:      0---255      0----100%

2. Total Strobe:      0---15 Open，16---255 Strobe from slow to fast

3. COB Bead effect:

0 No function, 1 red, 2 green, 3 blue, 4 white,	5 red + green,

6 red+ blue, 	7 red + white, 	8 green + blue, 9 green + white,

10 blue + white, 11 red + green + blue, 12 red + green + white,

13 red + blue + white, 14 green + blue + white,

15 red + green + blue + white，		16---20 effect 1 (Same direction)

21---25 effect 2 (Same direction) ，	26---30 effect 3 (Same direction)

31---35 effect 4 (Same direction) ，	36---39 effect 5 (Same direction)

40---44 effect 6 (Same direction) ， 	45---49 effect 7 (Same direction)

50---54effect 8 (Same direction) ， 	55---59 effect 9 (Same direction)

60---63 effect 10 (Same direction) ，64---68 effect 11 (Same direction)

69---73 effect 12 (Same direction) ，74---78 effect 13 (Same direction)

79---83 effect 14 (Same direction) ，84---87 effect 15 (Same direction)

88---92 effect 16 (Same direction) ，93---97effect 17 (Same direction)

98---102effect 18 (Same direction) ，103---107effect 19 (Same direction)

108---111effect 20 (Same direction)，112---116effect 21 (Same direction)

117---121effect 22 (Same direction)，122---126effect 23 (Same direction)

127---131effect 24 (Same direction) ，132---135effect 25 (Same direction)

136---140 effect 1(Reverse)     			141---145 effect 2(Reverse)

146---150 effect 3(Reverse)    		   151---155 effect 4(Reverse)

156---159 effect 5(Reverse)     			160---164 effect 6(Reverse)

165---169 effect 7(Reverse)    			170---174effect 8(Reverse)

175---179 effect 9(Reverse)     			180---183 effect 10(Reverse)

184---188 effect 11(Reverse)    			189---193 effect 12(Reverse)

194---198 effect 13(Reverse)    			199---203 effect 14(Reverse)

204---207 effect 15(Reverse)    			208---212 effect 16(Reverse)

213---217effect 17(Reverse)    			218---222effect 18(Reverse)

223---227effect 19(Reverse)    			228---231effect 20(Reverse)

232---236effect 21(Reverse)    			237---241effect 22(Reverse)

242---246effect 23(Reverse)    			247---251effect 24(Reverse)

252---255effect 25(Reverse)

4.  COB bead fixed color selection:

0  No function, 		1---255  15 color options available

5.  COB bead effect speed:  	0---255 	Speed from fast to slow

6.  5050 Light strip effect :  0 ---1 No function，2---3 Red，	4---5 Green，

6---7 Blue，8---9 red + green， 10---11 red + blue， 12---13 green + blue，

14---15 red + green + blue + white，		 16---20 effect 1 (Same direction),

21---25 effect 2 (Same direction),			 26---30 effect 3 (Same direction),

31---35 effect 4 (Same direction),			 36---39 effect 5 (Same direction)

40---44 effect 6 (Same direction)  		 45---49 effect 7 (Same direction)

50---54effect 8 (Same direction)   55---59 effect 9 (Same direction)

60---63 effect 10 (Same direction)  64---68 effect 11 (Same direction)

69---73 effect 12 (Same direction)  74---78 effect 13 (Same direction)

79---83 effect 14 (Same direction)  84---87 effect 15 (Same direction)

88---92 effect 16 (Same direction) 93---97effect 17 (Same direction)

98---102 effect 18 (Same direction)   103---107effect 19 (Same direction)    108---111effect 20 (Same direction)  112---116effect 21 (Same direction)     117---121effect 22 (Same direction)  122---126effect 23 (Same direction)     127---131effect 24 (Same direction)  			  132---135effect 25 (Same direction)

136---140 effect 1(Reverse)   		 	         141---145 effect 2(Reverse)

146---150 effect 3(Reverse)				      151---155 effect 4(Reverse)

156---159 effect 5(Reverse) 			 	      160---164 effect 6(Reverse)

165---169 effect 7(Reverse)     		 	      170---174effect 8(Reverse)

175---179 effect 9(Reverse)			 	      180---183 effect 10(Reverse)

184---188 effect 11(Reverse)	 		          189---193 effect 12(Reverse)

194---198 effect 13(Reverse) 			          199---203 effect 14(Reverse)

204---207 effect 15(Reverse)			          208---212 effect 16(Reverse)

213---217 effect 17(Reverse)    		          218---222effect 18(Reverse)

223---227 effect 19(Reverse) 			          228---231effect 20(Reverse)

232---236 effect 21(Reverse)			          237---241effect 22(Reverse)

242---246 effect 23(Reverse)	                 247---251effect 24(Reverse)

252---255effect 25(Reverse)

7. 5050 Light strip Fixed color selection: 0  No function, 1---255  7 color options

8.  5050 Light strip effect speed:   0---255 Speed from fast to slow

9.  Strobe Beads effect ：0---7 No function,	 	8---15  Full brightness

16---25 effect 1  		26---35 effect 2 			36---44 effect 3

45---54 effect 4			55---63 effect 5 			64---73 effect 6

74---83 effect 7 			84---92 effect 8			93---102 effect 9

103---111 effect 10		112---121 effect 11		122---131 effect 12		132---140 effect 13	   141---150 effect 14		151---159 effect 15		160---169 effect 16 	170---179 effect 17

180---18 8effect 18		189---198 effect 19 	199---207 effect 20		208---217effect 21

218---227effect 22 		228---236 effect 23		237---246effect 24		247---255effect 25

10.  Strobe Beads effect Speed: 	0---255 Speed changes from fast to slow

DMX 46 Channel function

1. Total dimming:       0---255      0----100% Dimming from dark to bright

2. Total Strobe:      	0---15 Open，16---255 Strobe from slow to fast

3. COB Beads effect :  0---15 No function	 16---20 effect 1 (Same direction)

21---25 effect 2 (Same direction)    26---30 effect 3 (Same direction)       31---35 effect 4 (Same direction)     36---39 effect 5 (Same direction)       40---44 effect 6 (Same direction)			  45---49 effect 7 (Same direction)

50---54 effect 8 (Same direction)			  55---59 effect 9 (Same direction)

60---63 effect 10 (Same direction)   		  64---68 effect 11 (Same direction)

69---73 effect 12 (Same direction) 		     74---78 effect 13 (Same direction)

79---83 effect 14 (Same direction)   		  84---87 effect 15 (Same direction)

88---92 effect 16 (Same direction)   		  93---97 effect 17 (Same direction)

98---102 effect 18 (Same direction) 	   103---107 effect 19 (Same direction)

108---111 effect 20 (Same direction) 	   112---116 effect 21 (Same direction)

117---121 effect 22 (Same direction)	   122---126 effect 23 (Same direction)

127---131 effect 24 (Same direction)	   132---135 effect 25 (Same direction)

136---140 effect 1(Reverse)     		   141---145 effect 2(Reverse)

146---150 effect 3(Reverse)     	    	151---155 effect 4(Reverse)

156---159 effect 5(Reverse)   		      160---164 effect 6(Reverse)

165---169 effect 7(Reverse)     		   170---174 effect 8(Reverse)

175---179 effect 9(Reverse)     		   180---183 effect 10(Reverse)

184---188 effect 11(Reverse)    		   189---193 effect 12(Reverse)

194---198 effect 13(Reverse)    		   199---203 effect 14(Reverse)

204---207 effect 15(Reverse)    		   208---212 effect 16(Reverse)

213---217 effect 17(Reverse)    		   218---222 effect 18(Reverse)

223---227 effect 19(Reverse)    		   228---231 effect 20(Reverse)

232---236 effect 21(Reverse)    		   237---241 effect 22(Reverse)

242---246 effect 23(Reverse)			   247---251 effect 24(Reverse)

252---255 effect 25(Reverse)

4. COB Color selection for lamp bead fixation:

0 No function，1---255   15 color options

5. COB Beads effect speed: 	0---255 Speed from fast to slow

6. COB Group 1 color mixing:	0---15 No function，	16---31 Red，

32---47 Green，48---63 Blue，64---79 White，80---95 Red + Green，96---111 Red + Blue，

112---127 Red + White，128---143 Green + Blue，144---159 Green + White，

160---175 Blue + White，176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

7. COB Group 2 color mixing:	0---15 No function，	16---31 Red，

32---47 Green，    	48---63 Blue，		64---79 White，80---95 Red + Green，		96---111 Red + Blue， 				112---127 Red + White，

128---143 Green + Blue，			144---159 Green + White，

160---175 Blue + White， 			176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

8. COB Group 3 color mixing:	0---15 No function，	16---31 Red，

32---47 Green，48---63 Blue，	64---79 White，80---95 Red + Green，

96---111 Red + Blue， 112---127 Red + White，128---143 Green + Blue，			144---159 Green + White，160---175 Blue + White，176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

9. COB Group 4 color mixing: 0---15 No function，	16---31 Red，

32---47 Green，       	48---63 Blue，			64---79 White，

80---95 Red + Green，		96---111 Red + Blue， 112---127 Red + White，

128---143 Green + Blue，			144---159 Green + White，

160---175 Blue + White， 			176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

10. COB Group 5 color mixing: 0---15 No function，	16---31 Red，

32---47 Green，       	48---63 Blue，			64---79 White，

80---95 Red + Green，		96---111 Red + Blue， 112---127 Red + White，

128---143 Green + Blue，			144---159 Green + White，

160---175 Blue + White， 			176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

11. COB Group 6 color mixing: 0---15 No function，	16---31 Red，

32---47 Green，       	48---63 Blue，			64---79 White，

80---95 Red + Green，		96---111 Red + Blue， 112---127 Red + White，

128---143 Green + Blue，			144---159 Green + White，

160---175 Blue + White， 			176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

12. COB Group 7 color mixing: 0---15 No function，	16---31 Red，

32---47 Green，       	48---63 Blue，			64---79 White，

80---95 Red + Green，		96---111 Red + Blue， 112---127 Red + White，

128---143 Green + Blue，			144---159 Green + White，

160---175 Blue + White， 			176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

13. COB Group 8 color mixing:	0---15 No function，	16---31 Red，

32---47 Green，       	48---63 Blue，			64---79 White，

80---95 Red + Green，		96---111 Red + Blue， 112---127 Red + White，

128---143 Green + Blue，			144---159 Green + White，

160---175 Blue + White， 			176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

14. COB Group 9 color mixing: 0---15 No function，	16---31 Red，

32---47 Green，       	48---63 Blue，			64---79 White，

80---95 Red + Green，		96---111 Red + Blue， 112---127 Red + White，

128---143 Green + Blue，			144---159 Green + White，

160---175 Blue + White， 			176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

15. COB Group 10 color mixing: 	0---15 No function，	16---31 Red，

32---47 Green，       	48---63 Blue，			64---79 White，

80---95 Red + Green，		96---111 Red + Blue， 112---127 Red + White，

128---143 Green + Blue，			144---159 Green + White，

160---175 Blue + White， 			176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

16. COB Group 11 color mixing: 0---15 No function，	16---31 Red，

32---47 Green，       	48---63 Blue，			64---79 White，

80---95 Red + Green，		96---111 Red + Blue， 112---127 Red + White，

128---143 Green + Blue，			144---159 Green + White，

160---175 Blue + White， 			176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

17. COB Group 12 color mixing: 0---15 No function，	16---31 Red，

32---47 Green，       	48---63 Blue，			64---79 White，

80---95 Red + Green，		96---111 Red + Blue， 112---127 Red + White，

128---143 Green + Blue，			144---159 Green + White，

160---175 Blue + White， 			176---191 Red + Green + Blue，

192---207 Red + Green + White，208---223 Red + Blue + White，

224---239 Green + Blue + White，240---255 Red + Green + Blue + White

18.  5050 Light strip effect :  			  0 ---15 No function

16---20 effect 1 (Same direction)      21---25 effect 2 (Same direction)

26---30 effect 3 (Same direction)      31---35 effect 4 (Same direction)

36---39 effect 5 (Same direction)      40---44 effect 6 (Same direction)

45---49 effect 7 (Same direction)      50---54effect 8 (Same direction)

55---59 effect 9 (Same direction)      60---63 effect 10 (Same direction)

64---68 effect 11 (Same direction)     69---73 effect 12 (Same direction)

74---78 effect 13 (Same direction)     79---83 effect 14 (Same direction)

84---87 effect 15 (Same direction)     88---92 effect 16 (Same direction)

93---97effect 17 (Same direction)      98---102effect 18 (Same direction)

103---107effect 19 (Same direction)   108---111effect 20 (Same direction)

112---116effect 21 (Same direction)   117---121effect 22 (Same direction)

122---126effect 23 (Same direction)   127---131effect 24 (Same direction)

132---135effect 25 (Same direction)	 136---140 effect 1(Reverse)

141---145 effect 2(Reverse				 146---150 effect 3(Reverse)

151---155 effect 4(Reverse)			 156---159 effect 5(Reverse)

160---164 effect 6(Reverse)			 165---169 effect 7(Reverse)

170---174effect 8(Reverse)				 175---179 effect 9(Reverse)

180---183 effect 10(Reverse)			 184---188 effect 11(Reverse)

189---193 effect 12(Reverse)	 		 194---198 effect 13(Reverse)

199---203 effect 14(Reverse)			 204---207 effect 15(Reverse)

208---212 effect 16(Reverse)			 213---217 effect 17(Reverse)

218---222effect 18(Reverse)			223---227effect 19(Reverse)

228---231effect 20(Reverse)			232---236effect 21(Reverse)

237---241effect 22(Reverse) 			242---246effect 23(Reverse)

247---251effect 24(Reverse)			252---255effect 25(Reverse)

19. 5050 Light strip  Fixed color selection:

0 	No function，1---255   7 color options

20. 5050 Light strip effect speed:   		0---255 Speed from fast to slow

21. 5050 Light strip Group 1 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

22. 5050 Light strip Group 2 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

23. 5050 Light strip Group 3 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

24. 5050 Light strip Group 4 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

25. 5050 Light strip Group 5 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

26. 5050 Light strip Group 6 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

27. 5050 Light strip Group 7 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

28. 5050 Light strip Group 8 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

29. 5050 Light strip Group 9 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

30. 5050 Light strip Group 10 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

31. 5050 Light strip Group 11 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

32. 5050 Light strip Group 12 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

33. 5050 Light strip Group 13 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

34. 5050 Light strip Group 14 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

35. 5050 Light strip Group 15 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

36. 5050 Light strip Group 16 color mixing：

0---31  No function 	32---63  Red  	64---95 Green  	96---127 Blue

28---159 Red + Green  160---191 Red + Blue	  192---223 Green + Blue

224---255 Red + Green+ Blue

37.  Strobe Beads effect ：

0---15  No function		16---25 effect 1        26---35 effect 2

36---44 effect 3        45---54 effect 4			 55---63 effect 5

64---73 effect 6			74---83 effect 7        84---92 effect 8

93---102 effect 9      103---111 effect 10		112---121 effect 11

122---131 effect 12		132---140 effect 13		141---150 effect 14

151---159 effect 15		160---169 effect 16		170---179 effect 17

180---188 effect 18		189---198 effect 19		199---207 effect 20

208---217 effect 21		218---227effect 22		228---236 effect 23

237---246 effect 24		247---255 effect 25

38. Strobe bead effect speed：0---255 Speed from fast to slow

39. Strobe Bead 1 Dimming：0---255   0----100% Dimming from dark to bright

40. Strobe Bead 2 Dimming：0---255   0----100% Dimming from dark to bright

41. Strobe Bead 3 Dimming：0---255   0----100% Dimming from dark to bright

42. Strobe Bead 4 Dimming：0---255   0----100% Dimming from dark to bright

43. Strobe Bead 5 Dimming：0---255   0----100% Dimming from dark to bright

44. Strobe Bead 6 Dimming：0---255   0----100% Dimming from dark to bright

45. Strobe Bead 7 Dimming：0---255   0----100% Dimming from dark to bright

46. Strobe Bead 8 Dimming：0---255   0----100% Dimming from dark to bright

DMX 119 Channel function

- Total dimming:       0---255      0----100% Dimming from dark to bright

- COB Beads Dimming：0---255      0----100% Dimming from dark to bright

- COB Beads Strobe：0---15 Open，16---255 Strobe from slow to fast

- 5050 Light strip  Dimming：0---255，0----100% Dimming from dark to bright

- 5050 Light strip strobe：0---15 Open，16---255 Strobe from slow to fast

- Strobe Beads Dimming：0---255，0----100% Dimming from dark to bright

- Strobe light bead strobe：0---15 Open，16---255 Strobe from slow to fast

8. COB Beads effect :  0 ---15 No function

16---20 effect 1 (Same direction) 21---25 effect 2(Same direction)         26---30 effect 3 (Same direction)       31---35 effect 4(Same direction)

36---39 effect 5 (Same direction)       40---44 effect 6(Same direction)         45---49 effect 7 (Same direction)       50---54effect 8(Same direction)         55---59 effect 9 (Same direction)       60---63 effect 10(Same direction)

64---68 effect 11 (Same direction)      69---73 effect 12(Same direction)

74---78 effect 13 (Same direction) 79---83 effect 14(Same direction)         84---87 effect 15 (Same direction)      88---92 effect 16(Same direction)

93---97effect 17 (Same direction)  98---102effect 18(Same direction)        103---107effect 19 (Same direction) 	  108---111effect 20(Same direction)

112---116effect 21 (Same direction)   117---121effect 22(Same direction)        122---126effect 23 (Same direction)   127---131effect 24(Same direction)        132---135effect 25(Same direction) 	  136---140 effect 1(Reverse)

141---145 effect 2(Reverse)         	  146---150 effect 3(Reverse)

151---155 effect 4(Reverse)         	  156---159 effect 5(Reverse)

160---164 effect 6(Reverse)     	     165---169 effect 7(Reverse)

170---174effect 8(Reverse)				  175---179 effect 9(Reverse)

180---183 effect 10(Reverse)			  184---188 effect 11(Reverse)

189---193 effect 12(Reverse)			  194---198 effect 13(Reverse)

199---203 effect 14(Reverse)   		  204---207 effect 15(Reverse)

208---212 effect 16(Reverse)   		  213---217effect 17(Reverse)

218---222effect 18(Reverse)			  223---227effect 19(Reverse)

228---231effect 20(Reverse)			  232---236effect 21(Reverse)

237---241effect 22(Reverse)     		  242---246effect 23(Reverse)

247---251effect 24(Reverse)        	  252---255effect 25(Reverse)

9. COB Beads Fixed color selection:  0 No function，1---255   15 color options

10. COB Beads effect speed:   0---255 Speed from fast to slow

11. COB Group 1 Red Dimming：0---255   0----100% Dimming from dark to bright

12. COB Group 1 Green Dimming：0---255   0----100% Dimming from dark to bright

13. COB Group 1 Blue Dimming：0---255   0----100% Dimming from dark to bright

14. COB Group 1 White Dimming：0---255   0----100% Dimming from dark to bright

15. COB Group 2 Red Dimming：0---255   0----100% Dimming from dark to bright

16. COB Group 2 Green Dimming：0---255   0----100% Dimming from dark to bright

17. COB Group 2 Blue Dimming：0---255   0----100% Dimming from dark to bright

18. COB Group 2 White Dimming：0---255   0----100% Dimming from dark to bright

19. COB Group 3 Red Dimming：0---255   0----100% Dimming from dark to bright

20. COB Group 3 Green Dimming：0---255   0----100% Dimming from dark to bright

21. COB Group 3 Blue Dimming：0---255   0----100% Dimming from dark to bright

22. COB Group 3 White Dimming：0---255   0----100% Dimming from dark to bright

23. COB Group 4 Red Dimming：0---255   0----100% Dimming from dark to bright

24. COB Group 4 Green Dimming：0---255   0----100% Dimming from dark to bright

25. COB Group 4 Blue Dimming：0---255   0----100% Dimming from dark to bright

26. COB Group 4 White Dimming：0---255   0----100% Dimming from dark to bright

27. COB Group 5 Red Dimming：0---255   0----100% Dimming from dark to bright

28. COB Group 5 Green Dimming：0---255   0----100% Dimming from dark to bright

29. COB Group 5 Blue Dimming：0---255   0----100% Dimming from dark to bright

30. COB Group 5 White Dimming：0---255   0----100% Dimming from dark to bright

31. COB Group 6 Red Dimming：0---255   0----100% Dimming from dark to bright

32. COB Group 6 Green Dimming：0---255   0----100% Dimming from dark to bright

33. COB Group 6 Blue Dimming：0---255   0----100% Dimming from dark to bright

34. COB Group 6 White Dimming：0---255   0----100% Dimming from dark to bright

35. COB Group 7 Red Dimming：0---255   0----100% Dimming from dark to bright

36. COB Group 7 Green Dimming：0---255   0----100% Dimming from dark to bright

37. COB Group 7 Blue Dimming：0---255   0----100% Dimming from dark to bright

38. COB Group 7 White Dimming：0---255   0----100% Dimming from dark to bright

39. COB Group 8 Red Dimming：0---255   0----100% Dimming from dark to bright

40. COB Group 8 Green Dimming：0---255   0----100% Dimming from dark to bright

41. COB Group 8 Blue Dimming：0---255   0----100% Dimming from dark to bright

42. COB Group 8 White Dimming：0---255   0----100% Dimming from dark to bright

43. COB Group 9 Red Dimming：0---255   0----100% Dimming from dark to bright

44. COB Group 9 Green Dimming：0---255   0----100% Dimming from dark to bright

45. COB Group 9 Blue Dimming：0---255   0----100% Dimming from dark to bright

46. COB Group 9 White Dimming：0---255   0----100% Dimming from dark to bright

47. COB Group 10 Red Dimming：0---255   0----100% Dimming from dark to bright

48. COB Group 10 Green Dimming：0---255   0----100% Dimming from dark to bright

49. COB Group 10 Blue Dimming：0---255   0----100% Dimming from dark to bright

50. COB Group 10 White Dimming：0---255   0----100% Dimming from dark to bright

51. COB Group 11 Red Dimming：0---255   0----100% Dimming from dark to bright

52. COB Group 11 Green Dimming：0---255   0----100% Dimming from dark to bright

53. COB Group 11 Blue Dimming：0---255   0----100% Dimming from dark to bright

54. COB Group 11 White Dimming：0---255   0----100% Dimming from dark to bright

55. COB Group 12 Red Dimming：0---255   0----100% Dimming from dark to bright

56. COB Group 12 Green Dimming：0---255   0----100% Dimming from dark to bright

57. COB Group 12 Blue Dimming：0---255   0----100% Dimming from dark to bright

58. COB Group 12 White Dimming：0---255   0----100% Dimming from dark to bright

59.  5050 Light strip effect :  0 ---15 No function

16---20 effect 1 (Same direction)       21---25 effect 2 (Same direction)         26---30 effect 3 (Same direction)       31---35 effect 4 (Same direction)

36---39 effect 5 (Same direction)     40---44 effect 6 (Same direction)         45---49 effect 7 (Same direction)   50---54effect 8 (Same direction)         55---59 effect 9 (Same direction)       60---63 effect 10 (Same direction)         64---68 effect 11 (Same direction)      69---73 effect 12 (Same direction)         74---78 effect 13 (Same direction)      79---83 effect 14 (Same direction)         84---87 effect 15 (Same direction)      88---92 effect 16 (Same direction)         93---97effect 17 (Same direction)      98---102effect 18 (Same direction)        103---107effect 19 (Same direction)     108---111effect 20 (Same direction)        112---116effect 21 (Same direction)     117---121effect 22 (Same direction)        122---126effect 23 (Same direction)     127---131effect 24 (Same direction)

132---135effect 25 (Same direction)         136---140 effect 1(Reverse)

141---145 effect 2(Reverse)                146---150 effect 3(Reverse)

151---155 effect 4(Reverse)     		      156---159 effect 5(Reverse)

160---164 effect 6(Reverse)					   165---169 effect 7(Reverse)     170---174effect 8(Reverse)						175---179 effect 9(Reverse)     180---183 effect 10(Reverse)				   184---188 effect 11(Reverse)    189---193 effect 12(Reverse)				   194---198 effect 13(Reverse)    199---203 effect 14(Reverse)				   204---207 effect 15(Reverse)    208---212 effect 16(Reverse)					213---217effect 17(Reverse) 18---222effect 18(Reverse)    223---227effect 19(Reverse)    228---231effect 20(Reverse)					232---236effect 21(Reverse)    237---241effect 22(Reverse)    242---246effect 23(Reverse)    247---251effect 24(Reverse)        252---255effect 25(Reverse)

60. 5050 Light strip  Fixed color selection:  0 No function，1---255   7 color options

61. 5050 Light strip effect speed:   0---255 Speed from fast to slow

62. 5050 Light strip Group 1 Red Dimming：0---255   0----100% Dimming from dark to bright

63. 5050 Light strip Group 1 Green Dimming：0---255   0----100% Dimming from dark to bright

64. 5050 Light strip Group 1 Blue Dimming：0---255   0----100% Dimming from dark to bright

65. 5050 Light strip Group 2 Red Dimming：0---255   0----100% Dimming from dark to bright

66. 5050 Light strip Group 2 Green Dimming：0---255   0----100% Dimming from dark to bright

67. 5050 Light strip Group 2 Blue Dimming：0---255   0----100% Dimming from dark to bright

68. 5050 Light strip Group 3 Red Dimming：0---255   0----100% Dimming from dark to bright

69. 5050 Light strip Group 3 Green Dimming：0---255   0----100% Dimming from dark to bright

70. 5050 Light strip Group 3 Blue Dimming：0---255   0----100% Dimming from dark to bright

71. 5050 Light strip Group 4 Red Dimming：0---255   0----100% Dimming from dark to bright

72. 5050 Light strip Group 4 Green Dimming：0---255   0----100% Dimming from dark to bright

73. 5050 Light strip Group 4 Blue Dimming：0---255   0----100% Dimming from dark to bright

74. 5050 Light strip Group 5 Red Dimming：0---255   0----100% Dimming from dark to bright

75. 5050 Light strip Group 5 Green Dimming：0---255   0----100% Dimming from dark to bright

76. 5050 Light strip Group 5 Blue Dimming：0---255   0----100% Dimming from dark to bright

77. 5050 Light strip Group 6 Red Dimming：0---255   0----100% Dimming from dark to bright

78. 5050 Light strip Group 6 Green Dimming：0---255   0----100% Dimming from dark to bright

79. 5050 Light strip Group 6 Blue Dimming：0---255   0----100% Dimming from dark to bright

80. 5050 Light strip Group 7 Red Dimming：0---255   0----100% Dimming from dark to bright

81. 5050 Light strip Group 7 Green Dimming：0---255   0----100% Dimming from dark to bright

82. 5050 Light strip Group 7 Blue Dimming：0---255   0----100% Dimming from dark to bright

83. 5050 Light strip Group 8 Red Dimming：0---255   0----100% Dimming from dark to bright

84. 5050 Light strip Group 8 Green Dimming：0---255   0----100% Dimming from dark to bright

85. 5050 Light strip Group 8 Blue Dimming：0---255   0----100% Dimming from dark to bright

86. 5050 Light strip Group 9 Red Dimming：0---255   0----100% Dimming from dark to bright

87. 5050 Light strip Group 9 Green Dimming：0---255   0----100% Dimming from dark to bright

88. 5050 Light strip Group 9 Blue Dimming：0---255   0----100% Dimming from dark to bright

89. 5050 Light strip Group 10 Red Dimming：0---255   0----100% Dimming from dark to bright

90. 5050 Light strip Group 10 Green Dimming：0---255   0----100% Dimming from dark to bright

91. 5050 Light strip Group 10 Blue Dimming：0---255   0----100% Dimming from dark to bright

92. 5050 Light strip Group 11 Red Dimming：0---255   0----100% Dimming from dark to bright

93. 5050 Light strip Group 11 Green Dimming：0---255   0----100% Dimming from dark to bright

94. 5050 Light strip Group 11 Blue Dimming：0---255   0----100% Dimming from dark to bright

95. 5050 Light strip Group 12 Red Dimming：0---255   0----100% Dimming from dark to bright

96. 5050 Light strip Group 12 Green Dimming：0---255   0----100% Dimming from dark to bright

97. 5050 Light strip Group 12 Blue Dimming：0---255   0----100% Dimming from dark to bright

98. 5050 Light strip Group 13 Red Dimming：0---255   0----100% Dimming from dark to bright

99. 5050 Light strip Group 13 Green Dimming：0---255   0----100% Dimming from dark to bright

100. 5050 Light strip Group 13 Blue Dimming：0---255   0----100% Dimming from dark to bright

101. 5050 Light strip Group 14 Red Dimming：0---255   0----100% Dimming from dark to bright

102. 5050 Light strip Group 14 Green Dimming：0---255   0----100% Dimming from dark to bright

103. 5050 Light strip Group 14 Blue Dimming：0---255   0----100% Dimming from dark to bright

104. 5050 Light strip Group 15 Red Dimming：0---255   0----100% Dimming from dark to bright

105. 5050 Light strip Group 15 Green Dimming：0---255   0----100% Dimming from dark to bright

106. 5050 Light strip Group 15 Blue Dimming：0---255   0----100% Dimming from dark to bright

107. 5050 Light strip Group 16 Red Dimming：0---255   0----100% Dimming from dark to bright

108. 5050 Light strip Group 16 Green Dimming：0---255   0----100% Dimming from dark to bright

109. 5050 Light strip Group 16 Blue Dimming：0---255   0----100% Dimming from dark to bright

110. Strobe Beads effect ：		0---15 No function		16---25 effect 1

26---35 effect 2				36---44 effect 3 			45---54 effect 4

55---63 effect 5				64---73 effect 6			74---83 effect 7

84---92effect 8				93---102 effect 9			103---111 effect 10

112---121 effect 11 			122---131 effect 12		132---140 effect 13

141---150 effect 14			151---159 effect 15		160---169 effect 16

170---179effect 17			180---188effect 18		189---198 effect 19

199---207effect 20			208---217effect 21		218---227effect 22

228---236 effect 23			237---246effect 24		247---255effect 25

111. Strobe Beads effect speed：0---255 Speed from fast to slow

112. Strobe Beads 1 Dimming:0---255   0----100% Dimming from dark to bright

113. Strobe Beads 2 Dimming:0---255   0----100% Dimming from dark to bright

114. Strobe Beads 3 Dimming:0---255   0----100% Dimming from dark to bright

115. Strobe Beads 4 Dimming:0---255   0----100% Dimming from dark to bright

116. Strobe Beads 5 Dimming:0---255   0----100% Dimming from dark to bright

117. Strobe Beads 6 Dimming:0---255   0----100% Dimming from dark to bright

118. Strobe Beads 7 Dimming: 0--255 0--100% Dimming from dark to bright

119. Strobe Beads 8 Dimming: 0--255 0--100% Dimming from dark to bright