60 LED RGBW

Par Light

☞ DMX MODE

4 Channels  Mode

| Channel | Function | Instruction |
| --- | --- | --- |
| CH1 | R Dimmer | Dimmer- Dark to Bright |
| CH2 | G Dimmer | Dimmer- Dark to Bright |
| CH3 | B Dimmer | Dimmer- Dark to Bright |
| CH4 | W Dimmer | Dimmer- Dark to Bright |

8 Channels  Mode

| Channel | Function | Instruction |
| --- | --- | --- |
| CH1 | Total Dimmer | RGBW Total Dimmer- Dark to Bright |
| CH2 | R Dimmer | Red Dimmer- Dark to Bright |
| CH3 | G Dimmer | Green Dimmer- Dark to Bright |
| CH4 | B Dimmer | Blue Dimmer- Dark to Bright |
| CH5 | W Dimmer | White Dimmer- Dark to Bright |
| CH6 | Total Strobe | RGBW Total Flash Strobe- Slow to Fast |
| CH7 | Function Choice | 000~050: CH1-CH6 Control. 051~100: Different Colors output 101~150: Colors Jump Change. 151~200: Colors Gradate. 201~250: Colors Pulse Change. 251~255: Sound-Active |
| CH8 | Function/Speed | Slow to Fast (At the Colors output choose the color) |

☞ KEYS INSTRUCTION

☞ DIGITAL SHOW FUNCTION FORM

| No | Show | Function | Value | Instruction |
| --- | --- | --- | --- | --- |
| 1 | A001 | DMX address | 001~512 | 8CH mode UP,DOWN increase or reduce the value |
| 2 | d001 | DMX address | 001~512 | 4CH mode,UP,DOWN increase or reduce the value, |
| 3 | r255 | Red dim | 000~255 | UP,DOWN Change brightness |
| 4 | G255 | Green dim | 000~255 | UP,DOWN Change brightness |
| 5 | b255 | Blue dim | 000~255 | UP,DOWN Change brightness |
| 6 | u255 | White dim | 000~255 | UP,DOWN Change brightness |
| 7 | F255 | Strobe | 010 ~ 255 | UP,DOWN change the speed |
| 8 | CC15 | Colors Jump | 00 ~ 15 | UP,DOWN change the speed |
| 9 | DE15 | Colors Gradate | 00 ~ 15 | UP,DOWN change the speed |
| 100 | CP15 | Colors Pulse | 00 ~ 15 | UP,DOWN change the speed |
| 11 | SoUd | Colors sound-Active | / | Sound-Active |