BEE EYE

LED1915

(Please read the instructions carefully and carefully before using the product)

Thank you for choosing to use our 1915 Bee's Eye light. For the correct and safe use of this product, please read the instructions carefully before installing and using this product. This specification contains important installation and application information, please strictly follow the instructions when installing and operating the product. At the same time, please keep this instruction manual properly.

Our 1915 Bee's Eye light uses a new and beautiful high-temperature resistant metal body. This product is designed and produced in strict accordance with CE standards, in line with the international standard DMX512 signal protocol, can be used alone for control, can also be used online, with fast rotation, low noise, powerful features, suitable for small and medium-sized concerts, theaters, studios, nightclubs and bars and other places.

Please remove the package carefully, check whether the product is damaged during transportation, and check whether the following contents are complete.

1915Bee's Eye light- - - - - - - - -1        instruction manual- - - - - - - - -1 book

Power line- - - - - - - -1,                  signal line- - - - - - - -1

This product is a good product before delivery. In order to keep the product in good condition and ensure safe operation, users should follow the safety matters and the instructions.

Important: Damage caused by not following this instruction is not covered by the warranty. The supplier is not responsible for the product problems arising from this.

If the product has been exposed to extreme unstable temperature conditions (e. g., after transportation), do not supply the product immediately because water drops due to temperature changes may damage the product. Please use the product has returned to normal temperature.

This product can be used in the voltage range of 90-240V, and is used for indoor products. Please ensure that the ground voltage is not higher than the tolerable range of the product!! The power plug must be plugged into a protective class I socket. Green or tea-cyan conductors must be grounded.

Linkage of the DMX512 signal

This lamp uses DMX512 signal control mode, and the control signal of each lamp is in parallel. When connecting the signals of multiple lamps, it is best to use dual-core shielding cable. When connecting, each lamp is connected through the DMX signal jack (Canon seat) INPUT (input) and OUTPUT (output). The 3-core XLR plug terminal of the signal line connecting the lamp must correspond to each other. When connecting the lamp signal, it is recommended to use the DMX signal terminal. To damaging the control signal due to electrical noise, the DMX signal terminal is a 120 ohm 1W resistance between the 2 and three feet of an XLR plug and connects it to the OUTPUT (output) jack of the last lamp.

Calculation method of lamp starting address code:

The starting address code of the current lamp is equal to (the starting address code of the previous lamp) + (number of channels of the lamp) description:

1: Start address code value of the first lamp A001.

2: The basic number of controller should be greater than or equal to the total number of lighting channels.

3: Note: when any controller is used, each lamp must have its own starting address code. If the starting address code of the first lamp is set A001, the pass number of the lamp is 16 CH; then the starting address code of the second lamp is set to A017; the starting address code of the third lamp is set to A033; and so on, (this setting mode should be determined by different console).

Instructions for lamp installation:

This lamp can be placed horizontally, oblique hanging and upside down, oblique hanging and upside down must pay attention to the installation method. Fixed installation of lamps: before the positioning of lamps, ensure the stability of the installation site, the lamps must not fall off the support frame, and use the support frame and the safety rope for auxiliary hanging; to ensure safety. Prevent the falling and sliding of the lamps and lanterns. During the installation and debugging, the pedestrians are prohibited to pass below, and regularly check whether the safety rope is worn and the hook screw is loose. If the installation of the lamp is not stable, the manufacturer will not bear any responsibility.

- ● Menu instructions

| primary menu | Sublevel menu | Three-level menu / parameters |
| --- | --- | --- |
| Address | 001 - 512 | The channel value is incremented or subtracted by 1 each time |
| System Settings | operational mode | Artnet/DMX / voice / self |
|  | channel pattern | 21CH/23CH/35CH/78CH/92CH/ 97CH/99CH |
|  | Default position of X axis | 0-255 |
|  | Default position of Y axis | 0-255 |
|  | X axis reversal | Open / close |
|  | Y axis reversal | Open / close |
|  | XY axis exchange | Open / close |
|  | XY axis fast mode | Open / close |
|  | The signal to keep | Open / close |
|  | Optical coupling error correction | Open / close |
|  | Hall error correction | Open / close |
|  | Synchronous update | Open / close |
|  | Backlit time | 15S/30S/60S/Often bright |
|  | Factory data reset | Confirm / cancel |
| Manual mode | Current channel mode channel | 0-255 |
| System calibration | Enter password | Lighting calibration |
| Advanced mode | Network |  |
|  | Data package |  |
|  | Head reset |  |
|  | XY reset |  |
|  | Full reset |  |
| System information | Reset the error message | Display the reset error message |
|  | DMX data monitoring | Channel values for the receiving console |
|  | Sensor information | Hall information |
|  |  | X-light coupling information |
|  |  | Y light coupling information |
|  | Hardware version | Display hardware version |
|  | software version | Displays the software version |
|  | Utility time | Displays the lamp service duration |

Screen automatic rotation function

The system can automatically rotate the screen according to the direction of gravity, without manual rotation. You can also turn off the automatic rotation function.

Manual control

This interface is used to control the current lamps.

Press OK to enter the edit state. Press Up and Down to change the channel value. Press OK again to save the modified value, exit editing, and press Exit not to save the modified value, and directly exit editing.

System calibration

Enter the password "0000" and press " OK to enter.

| option | illustrate |
| --- | --- |
| Motor calibration | After entering the sub-interface, the stroke of X axis, Y axis,rotation, focusing motor can be adjusted, and the adjustment range of 0~255,127 means that there is no adjustment. |
| Power calibration | After entering the sub-interface, the power of 19 beads can be adjusted, and the adjustment range of 0~255,255 means that there is no adjustment. |
| White balance calibration | After entering the sub-interface, the white balance calibration of the lamp’s total LED and LED1-19 can be adjusted, and the adjustment range of 0~255,255 means that there is no adjustment. |
| Other calibration | Voice-control sensitivity calibration and password modification. |

Reset

Press Up and Down to switch the reset mode, and press OK to reset directly.

| option | instruction |
| --- | --- |
| Network | IP/MS |
| Data package | Net(0-127)/Sub_Uni |
| Head reset | Effect motor reset except for XY |
| XY reset | XY axis reduction |
| Full reset | Lamps reset |

- system information

| option | instruction |
| --- | --- |
| Reset information | If the red ERR indicator shines, the lamp is running wrong, and details can be viewed in the subinterface |
| And DMX data monitoring | This enters the subinterface to display channel values for viewing |
| Sensor information | Real-time monitoring of the light coupling, hall and other sensors on the lamp state |
| Hardware version number | Lighting hardware information |
| Software version number | Lamp software version |

- ●Channels

(1)

| CH23 | CH99 | FUNCTION | DMX VALUE | FUNCTION DESCRIPTION |
| --- | --- | --- | --- | --- |
| 1 | 1 | Pan | 0-255 | X-axis control |
| 2 | 2 | Pan fine adjust | 0-255 | Pan fine adjust |
| 3 | 3 | Tilt | 0-255 | Y-axis control |
| 4 | 4 | Tilt fine adjust | 0-255 | Tilt fine adjust |
| 5 | 5 | Pan/Tilt speed | 0-255 | X-axis/Y-axis speed from fast to slow |
| 6 | 6 | Focus | 0-255 | Focus |
| 7 | 7 | Rotate | 0-127 | 0~60 degrees |
|  |  |  | 128-188 | Counterclockwise from fast to slow endless turn |
|  |  |  | 189-193 | Stop |
|  |  |  | 194-255 | Clockwise from slow to fast endless turn |
| 8 | 8 | Total dimmer | 0-255 | Linear dimming from dark to light |
| 9 | 9 | Stroboscopic | 0-3 | Light up |
|  |  |  | 4-200 | Synchronous stroboscopic speed from slow to fast |
|  |  |  | 201-215 | Low speed random stroboscopic |
|  |  |  | 216-234 | Medium-speed random stroboscopic |
|  |  |  | 235-255 | High-speed random stroboscopic |
| 10 | 10 | R-dimmer | 0-255 | Linear dimming from dark to light |
| 11 | 11 | G-dimmer | 0-255 | Linear dimming from dark to light |
| 12 | 12 | B-dimmer | 0-255 | Linear dimming from dark to light |
| 13 | 13 | W-dimmer | 0-255 | Linear dimming from dark to light |
| 14 | 14 | CTO | 0-255 | Linear color temperature regulation |
| 15 | 15 | Color macro | 0-255 | Built-in color gradient |
| 16 | 16 | Static effect | 0-9 | None |
|  |  |  | 10-14 | Effect 1 |
|  |  |  | 15-19 | Effect 2 |
|  |  |  | 20-24 | Effect 3 |
|  |  |  | ... |  |
|  |  |  | 5 Count an effect |  |
|  |  |  | ... |  |
|  |  |  | 175-179 | Effect 34 |
|  |  |  | 180-184 | Effect 35 |
|  |  |  | 185-255 | Effect 44 |
| 17 | 17 | Dynamic effect | 0-9 | None |
|  |  |  | 10-14 | Effect 1 |
|  |  |  | 15-19 | Effect 2 |
|  |  |  | 20-24 | Effect 3 |
|  |  |  | ... | ... |
|  |  |  | 5 Count an effect |  |
|  |  |  | 220-224 | Effect 43 |
|  |  |  | 225-229 | Effect 44 |
|  |  |  | 230-255 | Effect 45 |
| 18 | 18 | Dynamic effect speed | 0-126 | Positive effect from fast to slow |
|  |  |  | 127-128 | Stop |
|  |  |  | 129-255 | Reverse direction effect from slow to fast |
| 19 | 19 | Background color R | 0-255 | Linear dimming from dark to light |
| 20 | 20 | Background color G | 0-255 | Linear dimming from dark to light |
| 21 | 21 | Background color B | 0-255 | Linear dimming from dark to light |
| 22 | 22 | Background color W | 0-255 | Linear dimming from dark to light |
| 23 | 23 | Reset | 0-26 | None |
|  |  |  | 27-76 | Reset effect |
|  |  |  | 77-127 | Reset scan |
|  |  |  | 128-255 | Reset all |
| / | 24 | R1 LED dimming | 0-255 | R1 LED dimming |
|  | 25 | G1 LED dimming | 0-255 | G1 LED dimming |
|  | 26 | B1 LED dimming | 0-255 | B1 LED dimming |
|  | 27 | W1 LED dimming | 0-255 | W1 LED dimming |
|  | … | … | …… | …… |
|  | … | … | …… | …… |
|  | 96 | R19 LED dimming | 0-255 | R19 LED dimming |
|  | 97 | G19 LED dimming | 0-255 | G19 LED dimming |
|  | 98 | B19 LED dimming | 0-255 | B19 LED dimming |
|  | 99 | W19 LED dimming | 0-255 | W19 LED dimming |

- (2)

| CH21 | CH35 | CH78 | CH92 | CH97 | FUNCTION | DMX VALUE | FUNCTION DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | red | 0-255 | Linear dimming from dark to light |
| 2 | 2 | 2 | 2 | 2 | Red fine | 0-255 | Red fine |
| 3 | 3 | 3 | 3 | 3 | green | 0-255 | Linear dimming from dark to light |
| 4 | 4 | 4 | 4 | 4 | Green fine | 0-255 | Green fine |
| 5 | 5 | 5 | 5 | 5 | blue | 0-255 | Linear dimming from dark to light |
| 6 | 6 | 6 | 6 | 6 | Blue fine | 0-255 | Blue fine |
| 7 | 7 | 7 | 7 | 7 | white | 0-255 | Linear dimming from dark to light |
| 8 | 8 | 8 | 8 | 8 | White fine | 0-255 | White fine |
| 9 | 9 | 9 | 9 | 9 | CTO | 0-255 | CTO |
| 10 | 10 | 10 | 10 | 10 | Macro | 0-255 | Color macro |
| 11 | 11 | 11 | 11 | 11 | Strobe | 0-3 | Light off |
|  |  |  |  |  |  | 4-103 | Synchronous stroboscopic speed from slow to fast (1HZ-25HZ) |
|  |  |  |  |  |  | 104-107 | Light up |
|  |  |  |  |  |  | 108-207 | Bisecting strobe speed from slow to fast (1HZ-25HZ) |
|  |  |  |  |  |  | 208-212 | Light up |
|  |  |  |  |  |  | 213-225 | Low speed random stroboscopic |
|  |  |  |  |  |  | 226-238 | Medium-speed random stroboscopic |
|  |  |  |  |  |  | 239-251 | High-speed random stroboscopic |
|  |  |  |  |  |  | 252-255 | Light up |
| 12 | 12 | 12 | 12 | 12 | Dimmer | 0-255 | Linear dimming from dark to light |
| 13 | 13 | 13 | 13 | 13 | Dimmer fine | 0-255 | Dimmer fine |
| 14 | 14 | 14 | 14 | 14 | pan | 0-255 | X-axis control |
| 15 | 15 | 15 | 15 | 15 | Pan fine | 0-255 | Pan fine adjust |
| 16 | 16 | 16 | 16 | 16 | Tilt | 0-255 | Y-axis control |
| 17 | 17 | 17 | 17 | 17 | Tilt fine | 0-255 | Tilt fine adjust |
| 18 | 18 | 18 | 18 | 18 | Function | 0-255 | Reserve |
| 19 | 19 | 19 | 19 | 19 | Reset | 0-25 | None |
|  |  |  |  |  |  | 26-76 | Reset effect |
|  |  |  |  |  |  | 77-127 | Reset scan |
|  |  |  |  |  |  | 128-255 | Reset all |
| 20 | 20 | 20 | 20 | 20 | Focus | 0-255 | Focus |
| 21 | 21 | 21 | 21 | 21 | Rotate | 0-127 | 0~60 degrees |
|  |  |  |  |  |  | 128-188 | Counterclockwise from fast to slow endless turn |
|  |  |  |  |  |  | 189-193 | Stop |
|  |  |  |  |  |  | 194-255 | Clockwise from slow to fast endless turn |
| / | 22 | / | 22 | / | Built-in effect | 0-7 | None |
|  |  |  |  |  |  | 8 | Static effect1 |
|  |  |  |  |  |  | 9 | Static effect2 |
|  |  |  |  |  |  | 10 | Static effect3 |
|  |  |  |  |  |  | 11 | None |
|  |  |  |  |  |  | 12 | Static effect4 |
|  |  |  |  |  |  | 13 | Static effect5 |
|  |  |  |  |  |  | 14 | None |
|  |  |  |  |  |  | 15 | Dynamic effect1 |
|  |  |  |  |  |  | 16 | Dynamic effect2 |
|  |  |  |  |  |  | 17 | Dynamic effect3 |
|  |  |  |  |  |  | 18 | Dynamic effect4 |
|  |  |  |  |  |  | 19 | Monochromatic random |
|  |  |  |  |  |  | 20 | Full color random |
|  |  |  |  |  |  | 21 | Rainbow effect1 |
|  |  |  |  |  |  | 22 | Rainbow effect2 |
|  |  |  |  |  |  | 23 | Dynamic effect5 |
|  |  |  |  |  |  | 24 | Dynamic effect6 |
|  |  |  |  |  |  | 25 | Dynamic effect7 |
|  |  |  |  |  |  | 26 | Dynamic effect8 |
|  |  |  |  |  |  | 27 | Dynamic effect9 |
|  |  |  |  |  |  | 28 | Dynamic effect10 |
|  |  |  |  |  |  | 29 | Dynamic effect11 |
|  |  |  |  |  |  | 30 | Dynamic effect12 |
|  |  |  |  |  |  | 31 | Dynamic effect13 |
|  |  |  |  |  |  | 32 | Dynamic effect14 |
|  |  |  |  |  |  | 33 | None |
|  |  |  |  |  |  | 34 | None |
|  |  |  |  |  |  | 35 | Dynamic effect15 |
|  |  |  |  |  |  | 36 | Dynamic effect16 |
|  |  |  |  |  |  | 37 | Dynamic effect17 |
|  |  |  |  |  |  | 38 | Dynamic effect18 |
|  |  |  |  |  |  | 39 | Dynamic effect19 |
|  |  |  |  |  |  | 40 | Dynamic effect20 |
|  |  |  |  |  |  | 41 | None |
|  |  |  |  |  |  | 42 | Dynamic effect21 |
|  |  |  |  |  |  | 43 | None |
|  |  |  |  |  |  | 44 | Dynamic effect22 |
|  |  |  |  |  |  | 45 | Dynamic effect23 |
|  |  |  |  |  |  | 46 | Dynamic effect24 |
|  |  |  |  |  |  | 47 | Dynamic effect25 |
|  |  |  |  |  |  | 48 | Dynamic effect26 |
|  |  |  |  |  |  | 49 | Dynamic effect27 |
|  |  |  |  |  |  | 50 | Dynamic effect28 |
|  |  |  |  |  |  | 51 | Dynamic effect29 |
|  |  |  |  |  |  | 52 | Dynamic effect30 |
|  |  |  |  |  |  | 53 | Dynamic effect31 |
|  |  |  |  |  |  | 54 | Dynamic effect32 |
|  |  |  |  |  |  | 55 | Dynamic effect33 |
|  |  |  |  |  |  | 56 | Dynamic effect34 |
|  |  |  |  |  |  | 57 | Dynamic effect35 |
|  |  |  |  |  |  | 58 | Dynamic effect36 |
|  |  |  |  |  |  | 59 | Dynamic effect37 |
|  |  |  |  |  |  | 60 | Dynamic effect38 |
|  |  |  |  |  |  | 61 | Random position1 |
|  |  |  |  |  |  | 62 | Random position2 |
|  |  |  |  |  |  | 63 | Random position3 |
|  |  |  |  |  |  | 64 | None |
|  |  |  |  |  |  | 65 | Random position4 |
|  |  |  |  |  |  | 66 | Random position5 |
|  |  |  |  |  |  | 67 | Dynamic effect39 |
|  |  |  |  |  |  | 68 | Dynamic effect40 |
|  |  |  |  |  |  | 69 | None |
|  |  |  |  |  |  | 70 | Dynamic effect41 |
|  |  |  |  |  |  | 71-255 | None |
|  | 23 |  | 23 |  | Effect speed | 0-63 | Static effect |
|  |  |  |  |  |  | 64-158 | Forward from fast to slow |
|  |  |  |  |  |  | 159-160 | Stop |
|  |  |  |  |  |  | 161-255 | Reverse from slow to fast |
|  | 24 |  | 24 |  | Effect dilution | 0-255 | Dynamic effect dilution |
|  | 25 |  | 25 |  | Red effect | 0-255 | Red dynamic effect |
|  | 26 |  | 26 |  | Green effect | 0-255 | Green dynamic effect |
|  | 27 |  | 27 |  | Blue effect | 0-255 | Blue dynamic effect |
|  | 28 |  | 28 |  | White effect | 0-255 | White dynamic effect |
|  | 29 |  | 29 |  | Effect dimming | 0-255 | Pattern 0-100% dimming |
|  | 30 |  | 30 |  | Background dimming | 0-255 | Background 0-100% dimming |
|  | 31 |  | 31 |  | Effect transition | 0-255 | Pattern change control |
|  | 32 |  | 32 |  | Effect supplement | 0-255 | Pattern translation |
|  | 33 |  | 33 |  | Foreground stroboscopic | 0-255 | Foreground stroboscopic（same 11 channel） |
|  | 34 |  | 34 |  | Background stroboscopic | 0-255 | Background stroboscopic (same 11 channel) |
|  | 35 |  | 35 |  | Background selection | 0-255 | Background selection |
|  | / | 22 | 36 | 22 | R1 LED dimming | 0-255 | R1 LED dimming |
|  |  | 23 | 37 | 23 | G1 LED dimming | 0-255 | G1 LED dimming |
|  |  | 24 | 38 | 24 | B1 LED dimming | 0-255 | B1 LED dimming |
|  |  |  |  | 25 | W1 LED dimming | 0-255 | W1 LED dimming |
|  |  | 25 | 39 | 26 | R2 LED dimming | 0-255 | R2 LED dimming |
|  |  | … |  |  | … | 0-255 | …… |
|  |  | 76 | 91 | 94 | R19 LED dimming | 0-255 | R19 LED dimming |
|  |  | 77 | 90 | 95 | G19 LED dimming | 0-255 | G19 LED dimming |
|  |  | 78 | 92 | 96 | B19 LED dimming | 0-255 | B19 LED dimming |
|  |  | / | / | 97 | W19 LED dimming | 0-255 | W19 LED dimming |