Red, green, blue, amber, warm white five in one audience light with two eyes (1.3-inch OLED display)

Diagram section

When displayed normally

Button：

K1：Menu and Return

K2 & K3 : Adjust options and their numerical values

K4: Determine

When flipping the display

Button：

K3 : Menu and Return

K1&K4：Adjust options and their numerical values

K2: Determine

Attention:

- Lock screen password:

When displayed normally, K2-K3-K2-K3

When flipping the display, K3-K2-K3-K2

- When in the main interface, press the Determine key to enter the menu interface

- If the temperature control resistor is not successfully connected, the power of the lamp will be halved, and after 5 seconds, it will jump to the main interface and display an error message

- If there is no button operation within thirty seconds, the interface will jump to the main interface

- The Redshift only applies to the color temperature range of 2700K-3500K

Comparison Table of Functions Displayed on the Front Desk (bold font in the table represents the default values for this item)

Display window when language is set to English

| Main interface | Level 1 menu | Level 2 menu | Level 3 menu | Description |
| --- | --- | --- | --- | --- |
| When receiving a DMX signal, the screen displays "*". On this interface, the DMX address can be adjusted | DMX Mode | 1CH DWE |  |  |
|  |  | 2CH DWE |  |  |
|  |  | 4CH DWE |  |  |
|  |  | 9CH DWE+ |  |  |
|  |  | 6CH STROBE |  |  |
|  |  | 6CH RGB |  |  |
|  |  | 10CH BASIC RGB |  |  |
|  |  | 14CH BASIC RGB |  |  |
|  |  | 13CH STANDARD RGB |  |  |
|  |  | 22CH STANDARD RGB |  |  |
|  |  | 24CH STANDARD RGB |  |  |
|  |  | 33CH STANDARD RGB |  |  |
|  |  | 10CH DIRECT |  |  |
|  |  | 12CH DIRECT |  |  |
|  |  | 17CH DIRECT |  |  |
|  |  | 28CH DIRECT |  |  |
|  |  | 41CH DIRECT |  |  |
| The effect of execution can be adjusted on this interface | Auto | Dimmer 100 |  | 000-100 |
|  |  | Effect 00 |  | 00-10 00：Not executing the effect 01-10：Various effects |
|  |  | Speed 100 |  | 000-100 |
| On this interface, the overall dimming can be adjusted | Quick Color | Dimmer 100 |  | 000-100 |
|  |  | Shutter 255 |  | 000-255 (Refer to the flicker explanation of channel mode) |
|  |  | Red 000 |  | 000-255 |
|  |  | Green 000 |  | 000-255 |
|  |  | Blue 000 |  | 000-255 |
|  |  | Amber 000 |  | 000-255 |
|  |  | White 000 |  | 000-255 |
| On this interface, the color paper can be adjusted | Color Macro | Dimmer 100 |  | 000-100 |
|  |  | Color Macro 00 |  | 00-60 (For the corresponding names of colored paper, please refer to the colored paper table below) |
| Color temperature can be adjusted on this interface | Tunable White | Dimmer 100 |  | 000-100 |
|  |  | CCT 2900K |  | 2000K-10000K |
|  |  | Tint +000 |  | -127~+127 Negative values tend to be reddish, positive values tend to be greenish, and 0 represents neutral color |
| On this interface, you can select custom colors that have been programmed | User Color | Color 1 | Dimmer 100 | There are a total of five custom colors, and the operation items are shown in color 1 |
|  |  |  | Shutter 255 |  |
|  |  |  | Red 000 |  |
|  |  |  | Green 000 |  |
|  |  |  | Blue 000 |  |
|  |  |  | Amber 000 |  |
|  |  |  | White 000 |  |
|  |  | Color 2 |  |  |
|  |  | Color 3 |  |  |
|  |  | Color 4 |  |  |
|  |  | Color 5 |  |  |
|  | Settings | Language | 中文 | The screen display language is Chinese |
|  |  |  | English | The screen display language is English |
|  |  | MOSI Switch | Off | Do not send host data to slave |
|  |  |  | On | Send host data to slave |
|  |  | Screensaver | Off |  |
|  |  |  | On | Enter screensaver after 30 seconds without button operation |
|  |  | Screenflip | Off |  |
|  |  |  | On | The screen flips and the buttons also flip |
|  |  | Screenlock | Off |  |
|  |  |  | On | Lock after 30 seconds without button operation |
|  |  | DMX Fail | In DMX mode, the data status of the LED after signal interruption |  |
|  |  |  | Clear | The data will be cleared later |
|  |  |  | Hold | Data hold |
|  |  |  | Lit up | Keep the LED on at the power limit set by the current power setting |
|  |  | Dimmer Curve | Use the console to execute the power curve for regular dimming, without affecting the dimming, color temperature, color paper, and other modes with bulb effects (if the channel mode itself has a dimming curve, use the dimming curve channel as a guide) |  |
|  |  |  | Linear | Linear power curve |
|  |  |  | Square | Square power curve |
|  |  |  | Anti-square | Inverse square power curve |
|  |  |  | S-line | S power curve |
|  |  | Dimmer Bit | The process of changes in LED bead data |  |
|  |  |  | 8Bit | Direct switching of LED data |
|  |  |  | 16Bit | Smooth conversion of LED bead data |
|  |  | Dimmer Resp. | The speed of light changes (if the channel mode itself has a dimming speed, use the dimming speed channel as a guide) |  |
|  |  |  | Led |  |
|  |  |  | Medium |  |
|  |  |  | Halogen |  |
|  |  | LED Frequency | 1kHz |  |
|  |  |  | 2kHz |  |
|  |  |  | 4kHz |  |
|  |  |  | 12kHz |  |
|  |  |  | 16kHz |  |
|  |  |  | 25kHz |  |
|  |  | Redshift | Only applicable to color temperatures between 2700K-3500K |  |
|  |  |  | Off |  |
|  |  |  | On |  |
|  |  | Fan | Auto | If the temperature control resistor is not connected, the fan will rotate when the LED lights up. After connecting the temperature control resistor, when the temperature exceeds 44 ℃, the fan rotates; When the temperature drops below 40 ℃, the fan stops. The “automatic” speed can reach the maximum speed, while the “silent” speed is lower |
|  |  |  | Silent |  |
|  |  |  | Off |  |
|  |  |  | Max.Power |  |
|  |  | Invert Mapping | The direction of running and dimming of the lamp beads |  |
|  |  |  | Off |  |
|  |  |  | On |  |
|  |  | LED Mode | Set the power limit in the background to full power |  |
|  |  |  | Lower Power |  |
|  |  |  | High Power |  |
|  |  | Restore | Factory reset, backend parameters and language settings are not restored, and the interface jumps to the main interface |  |
|  |  |  | Off |  |
|  |  |  | On |  |
|  | System Info | Firmware VER | 250213D5V1 |  |
|  |  | RDM ID |  | Each device has a different ID number, consisting of 6 hexadecimal digits |
|  |  | Temperature | LED1：xx℃/xx℉ LED2：xx℃/xx℉ MAX：xx℃/xx℉ |  |
|  |  | Power on time | 0：00 |  |
|  |  | LED on time | LED1：0：00 LED2：0：00 |  |
|  |  | Fan state |  |  |
|  |  | Errors |  | If the temperature control resistor detection fails, the failed temperature control will be displayed |

Display window when language is set to Chinese

| Main interface | Level 1 menu | Level 2 menu | Level 3 menu | Description |
| --- | --- | --- | --- | --- |
| When receiving a DMX signal, the screen displays "*". On this interface, the DMX address can be adjusted | DMX模式 | 1CH DWE |  |  |
|  |  | 2CH DWE |  |  |
|  |  | 4CH DWE |  |  |
|  |  | 9CH DWE+ |  |  |
|  |  | 6CH STROBE |  |  |
|  |  | 6CH RGB |  |  |
|  |  | 10CH BASIC RGB |  |  |
|  |  | 14CH BASIC RGB |  |  |
|  |  | 13CH STANDARD RGB |  |  |
|  |  | 22CH STANDARD RGB |  |  |
|  |  | 24CH STANDARD RGB |  |  |
|  |  | 33CH STANDARD RGB |  |  |
|  |  | 10CH DIRECT |  |  |
|  |  | 12CH DIRECT |  |  |
|  |  | 17CH DIRECT |  |  |
|  |  | 28CH DIRECT |  |  |
|  |  | 41CH DIRECT |  |  |
| The effect of execution can be adjusted on this interface | 自动模式 | 总调光 100 |  | 000-100 |
|  |  | 效果 00 |  | 00-10 00：Not executing the effect 01-10：Various effects |
|  |  | 速度 100 |  | 000-100 |
| On this interface, the overall dimming can be adjusted | 手动调光 | 总调光 100 |  | 000-100 |
|  |  | 频闪 255 |  | 000-255 (Refer to the flicker explanation of channel mode) |
|  |  | 红色 000 |  | 000-255 |
|  |  | 绿色 000 |  | 000-255 |
|  |  | 蓝色 000 |  | 000-255 |
|  |  | 琥珀 000 |  | 000-255 |
|  |  | 暖白 000 |  | 000-255 |
| On this interface, the color paper can be adjusted | 色纸 | 总调光 100 |  | 000-100 |
|  |  | 色纸 00 |  | 00-60 (For the corresponding names of colored paper, please refer to the colored paper table below) |
| Color temperature can be adjusted on this interface | 色温 | 总调光 100 |  | 000-100 |
|  |  | 色温 2900K |  | 2000K-10000K |
|  |  | 色调 +000 |  | -127~+127 Negative values tend to be reddish, positive values tend to be greenish, and 0 represents neutral color |
| On this interface, you can select custom colors that have been programmed | 自定义颜色 | 颜色1 | 总调光 100 | There are a total of five custom colors, and the operation items are shown in color 1 |
|  |  |  | 频闪 255 |  |
|  |  |  | 红色 000 |  |
|  |  |  | 绿色 000 |  |
|  |  |  | 蓝色 000 |  |
|  |  |  | 琥珀 000 |  |
|  |  |  | 暖白 000 |  |
|  |  | 颜色2 |  |  |
|  |  | 颜色3 |  |  |
|  |  | 颜色4 |  |  |
|  |  | 颜色5 |  |  |
|  | 高级设置 | 语言 | 中文 | The screen display language is Chinese |
|  |  |  | English | The screen display language is English |
|  |  | 主从开关 | 关 | Do not send host data to slave |
|  |  |  | 开 | Send host data to slave |
|  |  | 屏保 | 关 |  |
|  |  |  | 开 | Enter screensaver after 30 seconds without button operation |
|  |  | 屏幕翻转 | 关 |  |
|  |  |  | 开 | The screen flips and the buttons also flip |
|  |  | 屏幕锁 | 关 |  |
|  |  |  | 开 | Lock after 30 seconds without button operation |
|  |  | DMX丢失 | In DMX mode, the data status of the LED after signal interruption |  |
|  |  |  | 清除 | The data will be cleared later |
|  |  |  | 保持 | Data hold |
|  |  |  | 亮灯 | Keep the LED on at the power limit set by the current power setting |
|  |  | 调光曲线 | Use the console to execute the power curve for regular dimming, without affecting the dimming, color temperature, color paper, and other modes with bulb effects (if the channel mode itself has a dimming curve, use the dimming curve channel as a guide) |  |
|  |  |  | 线性 | Linear power curve |
|  |  |  | 平方 | Square power curve |
|  |  |  | 反平方 | Inverse square power curve |
|  |  |  | S线 | S power curve |
|  |  | 调光位数 | The process of changes in LED bead data |  |
|  |  |  | 8位 | Direct switching of LED data |
|  |  |  | 16位 | Smooth conversion of LED bead data |
|  |  | 调光速度 | The speed of light changes (if the channel mode itself has a dimming speed, use the dimming speed channel as a guide) |  |
|  |  |  | 快速 |  |
|  |  |  | 中速 |  |
|  |  |  | 慢速 |  |
|  |  | 调光频率 | 1kHz |  |
|  |  |  | 2kHz |  |
|  |  |  | 4kHz |  |
|  |  |  | 12kHz |  |
|  |  |  | 16kHz |  |
|  |  |  | 25kHz |  |
|  |  | 灯泡效果 | Only applicable to color temperatures between 2700K-3500K |  |
|  |  |  | 关 |  |
|  |  |  | 开 |  |
|  |  | 风机模式 | 自动 | If the temperature control resistor is not connected, the fan will rotate when the LED lights up. After connecting the temperature control resistor, when the temperature exceeds 44 ℃, the fan rotates; When the temperature drops below 40 ℃, the fan stops. The “自动” speed can reach the maximum speed, while the “静默” speed is lower |
|  |  |  | 静默 |  |
|  |  |  | 关闭 |  |
|  |  |  | 最大功率 |  |
|  |  | 逆映射 | The direction of running and dimming of the lamp beads |  |
|  |  |  | 关 |  |
|  |  |  | 开 |  |
|  |  | LED模式 | Set the power limit in the background to full power |  |
|  |  |  | 半功率 |  |
|  |  |  | 全功率 |  |
|  |  | 恢复出厂 | Factory reset, backend parameters and language settings are not restored, and the interface jumps to the main interface |  |
|  |  |  | 关 |  |
|  |  |  | 开 |  |
|  | 系统信息 | 固件版本 | 250213D5V1 |  |
|  |  | RDM ID |  | Each device has a different ID number, consisting of 6 hexadecimal digits |
|  |  | 温度 | LED1：xx℃/xx℉ LED2：xx℃/xx℉ MAX：xx℃/xx℉ |  |
|  |  | 上电时间 | 0：00 |  |
|  |  | 亮灯时间 | LED1：0：00 LED2：0：00 |  |
|  |  | 风机状态 |  |  |
|  |  | 错误信息 |  | If the temperature control resistor detection fails, the failed temperature control will be displayed |

Comparison of backend display functions (bold font in the table represents the default values for this item)

When in the first level menu, adjust the cursor to the DMX item and long press the menu key to enter the background

Display window when language is set to English

| Menu item | Description |
| --- | --- |
| R current 207 | 020-255 |
| G current 217 | 020-255 |
| B current 212 | 020-255 |
| A current 219 | 020-255 |
| W current 222 | 020-255 |
| P limit 180 | 050-255 |
| T1 protect 70 | 40-80 When the temperature of the lamp body exceeds the set value, the power of the lamp bead decreases Attention: The lamp body needs to be connected to a temperature control resistor before this function can be activated |
| T2 protect 70 | 40-80 When the temperature of the lamp body exceeds the set value, the power of the lamp bead decreases Attention: The lamp body needs to be connected to a temperature control resistor before this function can be activated |
| Push Shut down | Push the backend parameters of the local machine, set parameters and channel modes to the slave machine. Shut down：Do not push Turn on：Push Attention: Please ensure that the signal line between the host and the slave is connected and powered on, and that the slave display interface is a DMX address interface. When pushing, wait for the LED of the slave device to flash (indicating that the data sent by the host has been received and saved) before the host can exit the push state. |

Display window when language is set to Chinese

| Menu item | Description |
| --- | --- |
| 红色电流 207 | 020-255 |
| 绿色电流 217 | 020-255 |
| 蓝色电流 212 | 020-255 |
| 琥珀色电流 219 | 020-255 |
| 暖白色电流 222 | 020-255 |
| 功率限制 180 | 050-255 |
| 温度1保护 70 | 40-80 When the temperature of the lamp body exceeds the set value, the power of the lamp bead decreases Attention: The lamp body needs to be connected to a temperature control resistor before this function can be activated |
| 温度2保护 70 | 40-80 When the temperature of the lamp body exceeds the set value, the power of the lamp bead decreases Attention: The lamp body needs to be connected to a temperature control resistor before this function can be activated |
| 推送 关闭 | Push the backend parameters of the local machine, set parameters and channel modes to the slave machine. Shut down：Do not push Turn on：Push Attention: Please ensure that the signal line between the host and the slave is connected and powered on, and that the slave display interface is a DMX address interface. When pushing, wait for the LED of the slave device to flash (indicating that the data sent by the host has been received and saved) before the host can exit the push state. |

DMX channel table

| CH | 1CH DWE | 2CH DWE | 4CH DWE | 9CH DWE+ | 6CH STROBE |
| --- | --- | --- | --- | --- | --- |
| 1 | Dimmer | Dimmer1 | Dimmer 1 | Master Dimmer | Master Dimmer |
| 2 |  | Dimmer2 | Dimmer Curve | Shutter | Shutter |
| 3 |  |  | Dimmer Response | Red | Duration |
| 4 |  |  | Redshift | Green | Dimmer 1 |
| 5 |  |  |  | Blue | Dimmer 2 |
| 6 |  |  |  | CTC | CTC |
| 7 |  |  |  | Dimmer Curve |  |
| 8 |  |  |  | Dimmer Response |  |
| 9 |  |  |  | Redshift |  |

| CH | 6CH RGB | 10CH BASIC RGB | 14CH BASIC RGB | 13CH STANDARD RGB |
| --- | --- | --- | --- | --- |
| 1 | Red 1 | Master Dimmer | Master Dimmer | Master Dimmer |
| 2 | Green 1 | Shutter | Master Dimmer Fine | Shutter |
| 3 | Blue 1 | Red | Shutter | Dimmer 1 |
| 4 | Red 2 | Green | Red | Red 1 |
| 5 | Green 2 | Blue | Red Fine | Green 1 |
| 6 | Blue 2 | CTC | Green | Blue 1 |
| 7 |  | Tint | Green Fine | Dimmer 2 |
| 8 |  | Color Macro | Blue | Red 2 |
| 9 |  | Color Macro Crossfade | Blue Fine | Green 2 |
| 10 |  | Device Settings | CTC | Blue 2 |
| 11 |  |  | Tint | CTC (all) |
| 12 |  |  | Color Marco | Tint (all) |
| 13 |  |  | Color Macro Crossfade | Device Settings |
| 14 |  |  | Device Settings |  |

| CH | 22CH STANDARD RGB | 24CH EXTENDED RGB | 33CH EXTENDED RGB |
| --- | --- | --- | --- |
| 1 | Master Dimmer | Master Dimmer | Master Dimmer |
| 2 | Master Dimmer Fine | Shutter | Master Dimmer Fine |
| 3 | Shutter | Duration | Shutter |
| 4 | Dimmer 1 | Dimmer 1 | Duration |
| 5 | Dimmer 1 Fine | Red 1 | Dimmer 1 |
| 6 | Red 1 | Green 1 | Dimmer 1 Fine |
| 7 | Red 1 Fine | Blue 1 | Red 1 |
| 8 | Green 1 | CTC 1 | Red 1 Fine |
| 9 | Green 1 Fine | Tint 1 | Green 1 |
| 10 | Blue 1 | Color Macro 1 | Green 1 Fine |
| 11 | Blue 1 Fine | Color Macro Crossfade 1 | Blue 1 |
| 12 | Dimmer 2 | Dimmer 2 | Blue 1 Fine |
| 13 | Dimmer 2 Fine | Red 2 | CTC 1 |
| 14 | Red 2 | Green 2 | Tint 1 |
| 15 | Red 2 Fine | Blue 2 | Color Macro 1 |
| 16 | Green 2 | CTC 2 | Color Macro Crossfade 1 |
| 17 | Green 2 Fine | Tint 2 | Dimmer 2 |
| 18 | Blue 2 | Color Macro 2 | Dimmer 2 Fine |
| 19 | Blue 2 Fine | Color Macro Crossfade 2 | Red 2 |
| 20 | CTC (all) | Effect | Red 2 Fine |
| 21 | Tint (all) | Dimmer Curve | Green 2 |
| 22 | Device Settings | Dimmer Response | Green 2 Fine |
| 23 |  | Redshift | Blue 2 |
| 24 |  | Device Settings | Blue 2 Fine |
| 25 |  |  | CTC 2 |
| 26 |  |  | Tint 2 |
| 27 |  |  | Color Macro 2 |
| 28 |  |  | Color Macro Crossfade 2 |
| 29 |  |  | Effect |
| 30 |  |  | Dimmer Curve |
| 31 |  |  | Dimmer Response |
| 32 |  |  | Redshift |
| 33 |  |  | Device Settings |

| CH | 10CH DIRECT | 12CH DIRECT | 17CH DIRECT | 28CH DIRECT | 41CH DIRECT |
| --- | --- | --- | --- | --- | --- |
| 1 | Red 1 | Master Dimmer | Master Dimmer | Master Dimmer | Master Dimmer |
| 2 | Green 1 | Shutter | Shutter | Shutter | Master Dimmer Fine |
| 3 | Blue 1 | Red | Dimmer 1 | Duration | Shutter |
| 4 | Amber 1 | Green | Red 1 | Dimmer 1 | Duration |
| 5 | White 1 | Blue | Green 1 | Red 1 | Dimmer 1 |
| 6 | Red 2 | Amber | Blue 1 | Green 1 | Dimmer 1 Fine |
| 7 | Green 2 | White | Amber 1 | Blue 1 | Red 1 |
| 8 | Blue 2 | CTC | White 1 | Amber 1 | Red 1 Fine |
| 9 | Amber 2 | Tint | Dimmer 2 | White 1 | Green 1 |
| 10 | White 2 | Color Macro | Red 2 | CTC 1 | Green 1 Fine |
| 11 |  | Color Macro Crossfade | Green 2 | Tint 1 | Blue 1 |
| 12 |  | Device Settings | Blue 2 | Color Macro 1 | Blue 1 Fine |
| 13 |  |  | Amber 2 | Color Macro Crossfade 1 | Amber 1 |
| 14 |  |  | White 2 | Dimmer 2 | Amber 1 Fine |
| 15 |  |  | CTC (all) | Red 2 | White 1 |
| 16 |  |  | Tint (all) | Green 2 | White 1 Fine |
| 17 |  |  | Device Settings | Blue 2 | CTC 1 |
| 18 |  |  |  | Amber 2 | Tint 1 |
| 19 |  |  |  | White 2 | Color Macro 1 |
| 20 |  |  |  | CTC 2 | Color Macro Crossfade 1 |
| 21 |  |  |  | Tint 2 | Dimmer 2 |
| 22 |  |  |  | Color Macro 2 | Dimmer 2 Fine |
| 23 |  |  |  | Color Macro Crossfade 2 | Red 2 |
| 24 |  |  |  | Effect | Red 2 Fine |
| 25 |  |  |  | Dimmer Curve | Green 2 |
| 26 |  |  |  | Dimmer Response | Green 2 Fine |
| 27 |  |  |  | Redshift | Blue 2 |
| 28 |  |  |  | Device Settings | Blue 2 Fine |
| 29 |  |  |  |  | Amber 2 |
| 30 |  |  |  |  | Amber 2 Fine |
| 31 |  |  |  |  | White 2 |
| 32 |  |  |  |  | White 2 Fine |
| 33 |  |  |  |  | CTC 2 |
| 34 |  |  |  |  | Tint 2 |
| 35 |  |  |  |  | Color Macro 2 |
| 36 |  |  |  |  | Color Macro Crossfade 2 |
| 37 |  |  |  |  | Effect |
| 38 |  |  |  |  | Dimmer Curve |
| 39 |  |  |  |  | Dimmer Response |
| 40 |  |  |  |  | Redshift |
| 41 |  |  |  |  | Device Settings |

1CH DWE Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer | Manifested as the current color temperature in color temperature mode |

2CH DWE Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer 1 | Manifested as the current color temperature in color temperature mode |
| 2 | Dimmer 2 | Manifested as the current color temperature in color temperature mode |

4CH DWE Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer | Manifested as the current color temperature in color temperature mode |
| 2 | Dimmer Curve | 0-51，Current Preset from Menu Settings 52-101，Linear dimmer curve 102-152，Square dimmer curve 153-203，Anti-square dimmer curve 204-255，S-line dimmer curve |
| 3 | Dimmer Response | 0-63，Current Preset from Menu Settings 64-127，LED/fast 128-191，Medium 192-255，Halogen/slow |
| 4 | Redshift | 0-84，Current Preset from Menu Settings 85-170，Off 171-255，On |

9CH DWE+ Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 3 | R Dimmer |  |
| 4 | G Dimmer |  |
| 5 | B Dimmer |  |
| 6 | CCT(affects RGB) | 000-004,2900K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 7 | Dimmer Curve | 0-51，Current Preset from Menu Settings 52-101，Linear dimmer curve 102-152，Square dimmer curve 153-203，Anti-square dimmer curve 204-255，S-line dimmer curve |
| 8 | Dimmer Response | 0-63，Current Preset from Menu Settings 64-127，LED/fast 128-191，Medium 192-255，Halogen/slow |
| 9 | Redshift | 0-84，Current Preset from Menu Settings 85-170，Off 171-255，On |

6CH STROBE Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 3 | Duration | Extend the lighting time during flicker, only applicable to the normal flicker of channel 2（025-064） |
| 4 | Dimmer 1 |  |
| 5 | Dimmer 2 |  |
| 6 | CCT | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |

6CH RGB Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | R1 dimmer |  |
| 2 | G1 dimmer |  |
| 3 | B1 dimmer |  |
| 4 | R2 dimmer |  |
| 5 | G2 dimmer |  |
| 6 | B2 dimmer |  |

10CH BASIC RGB Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 3 | R dimmer |  |
| 4 | G dimmer |  |
| 5 | B dimmer |  |
| 6 | CCT(affects RGB) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 7 | Tint（affects CCT and RGB） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 8 | Color Macro（override RGB/CCT） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 9 | Color Macro Cross fade (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 10 | Device Settings | The push rod needs to be pushed for the first time to cause a change in the value before it can be considered as a setting. The following functions require a 3-second pause to take effect 000-029 No function 030-034 8bit 035-039 16bit 040-044 No function 045-049 DMX Fail Blackout 050-054 DMX Fail Hold 055-059 DMX Fail - Emergency Light 060-064 No function 065-069 Linear dimming curve 070-074 Square dimming curve 075-079 Anti-square dimming curve 080-084 S dimming curve 085-089 No function 090-094 Dimmer Response fast 095-099 Dimmer Response medium 100-104 Dimmer Response slow(halogen) 105-109 No function 110-114 Redshift on（affects only between 2700-3500K） 115-119 Redshift off 120-139 No function 140-144 LED Frequency 1kHz 145-149 LED Frequency 2kHz 150-154 LED Frequency 4kHz 155-159 LED Frequency 12kHz 160-164 LED Frequency 16kHz 165-169 LED Frequency 25kHz 170-174 No function 175-179 Fan Auto 180-184 Fan Silent 185-189 Fan Off 190-194 Fan High Power 195-199 No function 200-214 Invert Mapping Off 215-219 Invert Mapping On 220-224 No function 225-229 LED Mode - low-power 230-234 LED Mode - High-power 235-239 No function 240-255 The current setting is invalid |

14CH BASIC RGB Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Dimmer fine-tuning |  |
| 3 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 4 | R dimmer |  |
| 5 | R dimmer fine-tuning |  |
| 6 | G dimmer |  |
| 7 | G dimmer fine-tuning |  |
| 8 | B dimmer |  |
| 9 | B dimmer fine-tuning |  |
| 10 | CCT(affects RGB) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 11 | Tint（affects CCT and RGB） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 12 | Color Macro（override RGB/CCT） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 13 | Color Macro Cross fade (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 14 | Device Settings | The push rod needs to be pushed for the first time to cause a change in the value before it can be considered as a setting. The following functions require a 3-second pause to take effect 000-029 No function 030-034 8bit 035-039 16bit 040-044 No function 045-049 DMX Fail Blackout 050-054 DMX Fail Hold 055-059 DMX Fail - Emergency Light 060-064 No function 065-069 Linear dimming curve 070-074 Square dimming curve 075-079 Anti-square dimming curve 080-084 S dimming curve 085-089 No function 090-094 Dimmer Response fast 095-099 Dimmer Response medium 100-104 Dimmer Response slow(halogen) 105-109 No function 110-114 Redshift on（affects only between 2700-3500K） 115-119 Redshift off 120-139 No function 140-144 LED Frequency 1kHz 145-149 LED Frequency 2kHz 150-154 LED Frequency 4kHz 155-159 LED Frequency 12kHz 160-164 LED Frequency 16kHz 165-169 LED Frequency 25kHz 170-174 No function 175-179 Fan Auto 180-184 Fan Silent 185-189 Fan Off 190-194 Fan High Power 195-199 No function 200-214 Invert Mapping Off 215-219 Invert Mapping On 220-224 No function 225-229 LED Mode - low-power 230-234 LED Mode - High-power 235-239 No function 240-255 The current setting is invalid |

13CH STANDARD RGB Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 3 | Dimmer 1 |  |
| 4 | R1 dimmer |  |
| 5 | G1 dimmer |  |
| 6 | B1 dimmer |  |
| 7 | Dimmer 2 |  |
| 8 | R2 dimmer |  |
| 9 | G2 dimmer |  |
| 10 | B2 dimmer |  |
| 11 | CCT（affects RGB1+2) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 12 | Tint（affects CCT and RGB 1+2） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 13 | Device Settings | The push rod needs to be pushed for the first time to cause a change in the value before it can be considered as a setting. The following functions require a 3-second pause to take effect 000-029 No function 030-034 8bit 035-039 16bit 040-044 No function 045-049 DMX Fail Blackout 050-054 DMX Fail Hold 055-059 DMX Fail - Emergency Light 060-064 No function 065-069 Linear dimming curve 070-074 Square dimming curve 075-079 Anti-square dimming curve 080-084 S dimming curve 085-089 No function 090-094 Dimmer Response fast 095-099 Dimmer Response medium 100-104 Dimmer Response slow(halogen) 105-109 No function 110-114 Redshift on（affects only between 2700-3500K） 115-119 Redshift off 120-139 No function 140-144 LED Frequency 1kHz 145-149 LED Frequency 2kHz 150-154 LED Frequency 4kHz 155-159 LED Frequency 12kHz 160-164 LED Frequency 16kHz 165-169 LED Frequency 25kHz 170-174 No function 175-179 Fan Auto 180-184 Fan Silent 185-189 Fan Off 190-194 Fan High Power 195-199 No function 200-214 Invert Mapping Off 215-219 Invert Mapping On 220-224 No function 225-229 LED Mode - low-power 230-234 LED Mode - High-power 235-239 No function 240-255 The current setting is invalid |

22CH STANDARD RGB Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Dimmer fine-tuning |  |
| 3 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 4 | Dimmer 1 |  |
| 5 | Dimmer 1 fine-tuning |  |
| 6 | R1 dimmer |  |
| 7 | R1 dimmer fine-tuning |  |
| 8 | G1 dimmer |  |
| 9 | G1 dimmer fine-tuning |  |
| 10 | B1 dimmer |  |
| 11 | B1 dimmer fine-tuning |  |
| 12 | Dimmer 2 |  |
| 13 | Dimmer 2 fine-tuning |  |
| 14 | R1 dimmer |  |
| 15 | R1 dimmer fine-tuning |  |
| 16 | G2 dimmer |  |
| 17 | G2 dimmer fine-tuning |  |
| 18 | B2 dimmer |  |
| 19 | B2 dimmer fine-tuning |  |
| 20 | CCT(affects RGB 1+2) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 21 | Tint（affects CCT and RGB 1+2） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 22 | Device Settings | The push rod needs to be pushed for the first time to cause a change in the value before it can be considered as a setting. The following functions require a 3-second pause to take effect 000-029 No function 030-034 8bit 035-039 16bit 040-044 No function 045-049 DMX Fail Blackout 050-054 DMX Fail Hold 055-059 DMX Fail - Emergency Light 060-064 No function 065-069 Linear dimming curve 070-074 Square dimming curve 075-079 Anti-square dimming curve 080-084 S dimming curve 085-089 No function 090-094 Dimmer Response fast 095-099 Dimmer Response medium 100-104 Dimmer Response slow(halogen) 105-109 No function 110-114 Redshift on（affects only between 2700-3500K） 115-119 Redshift off 120-139 No function 140-144 LED Frequency 1kHz 145-149 LED Frequency 2kHz 150-154 LED Frequency 4kHz 155-159 LED Frequency 12kHz 160-164 LED Frequency 16kHz 165-169 LED Frequency 25kHz 170-174 No function 175-179 Fan Auto 180-184 Fan Silent 185-189 Fan Off 190-194 Fan High Power 195-199 No function 200-214 Invert Mapping Off 215-219 Invert Mapping On 220-224 No function 225-229 LED Mode - low-power 230-234 LED Mode - High-power 235-239 No function 240-255 The current setting is invalid |

24CH EXTENDED RGB Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 3 | Duration | Extend the lighting time during flicker, only applicable to the normal flicker of channel 2（025-064） |
| 4 | Dimmer 1 |  |
| 5 | R1 dimmer |  |
| 6 | G1 dimmer |  |
| 7 | B1 dimmer |  |
| 8 | CCT 1(affects RGB 1) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 9 | Tint 1（affects CCT and RGB 1） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 10 | Color Macro 1（override RGB /CCT 1） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 11 | Color Macro Cross Fade 1 (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 12 | Dimmer 2 |  |
| 13 | R2 dimmer |  |
| 14 | G2 dimmer |  |
| 15 | B2 dimmer |  |
| 16 | CCT 2(affects RGB 2) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 17 | Tint 2（affects CCT and RGB 2） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 18 | Color Macro 2（override RGB /CCT 2） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 19 | Color Macro Cross Fade 2 (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 20 | Effect（override RGB 1+2/CCT 1+2/Color Macro 1+2） | 000-015，empty 016-039，effect 1（from slow to fast） 040-063，effect 2（from slow to fast） 064-087，effect 3（from slow to fast） 088-111，effect 4（from slow to fast） 112-135，effect 5（from slow to fast） 136-159，effect 6（from slow to fast） 160-183，effect 7（from slow to fast） 184-207，effect 8（from slow to fast） 208-231，effect 9（from slow to fast） 232-255，effect 10（from slow to fast） |
| 21 | Dimmer Curve | 0-51，Current Preset from Menu Settings 52-101，Linear dimmer curve 102-152，Square dimmer curve 153-203，Anti-square dimmer curve 204-255，S-line dimmer curve |
| 22 | Dimmer Response | 0-63，Current Preset from Menu Settings 64-127，LED/fast 128-191，Medium 192-255，Halogen/slow |
| 23 | Redshift | 0-84，Current Preset from Menu Settings 85-170，Off 171-255，On |
| 24 | Device Settings | The push rod needs to be pushed for the first time to cause a change in the value before it can be considered as a setting. The following functions require a 3-second pause to take effect 000-029 No function 030-034 8bit 035-039 16bit 040-044 No function 045-049 DMX Fail Blackout 050-054 DMX Fail Hold 055-059 DMX Fail - Emergency Light 060-064 No function 065-069 Linear dimming curve 070-074 Square dimming curve 075-079 Anti-square dimming curve 080-084 S dimming curve 085-089 No function 090-094 Dimmer Response fast 095-099 Dimmer Response medium 100-104 Dimmer Response slow(halogen) 105-109 No function 110-114 Redshift on（affects only between 2700-3500K） 115-119 Redshift off 120-139 No function 140-144 LED Frequency 1kHz 145-149 LED Frequency 2kHz 150-154 LED Frequency 4kHz 155-159 LED Frequency 12kHz 160-164 LED Frequency 16kHz 165-169 LED Frequency 25kHz 170-174 No function 175-179 Fan Auto 180-184 Fan Silent 185-189 Fan Off 190-194 Fan High Power 195-199 No function 200-214 Invert Mapping Off 215-219 Invert Mapping On 220-224 No function 225-229 LED Mode - low-power 230-234 LED Mode - High-power 235-239 No function 240-255 The current setting is invalid |

33CH EXTENDED RGB Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Dimmer fine-tuning |  |
| 3 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 4 | 持续时间 | 延长频闪时的亮灯时间，仅作用于通道2的普通频闪（025-064） |
| 5 | Dimmer 1 |  |
| 6 | Dimmer 1 fine-tuning |  |
| 7 | R1 dimmer |  |
| 8 | R1 dimmer fine-tuning |  |
| 9 | G1 dimmer |  |
| 10 | G1 dimmer fine-tuning |  |
| 11 | B1 dimmer |  |
| 12 | B1 dimmer fine-tuning |  |
| 13 | CCT 1(affects RGB 1) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 14 | Tint 1（affects CCT and RGB 1） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 15 | Color Macro 1（override RGB /CCT 1） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 16 | Color Macro Cross Fade 1 (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 17 | Dimmer 2 |  |
| 18 | Dimmer 2 fine-tuning |  |
| 19 | R1 dimmer |  |
| 20 | R1 dimmer fine-tuning |  |
| 21 | G2 dimmer |  |
| 22 | G2 dimmer fine-tuning |  |
| 23 | B2 dimmer |  |
| 24 | B2 dimmer fine-tuning |  |
| 25 | CCT 2(affects RGB 2) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 26 | Tint 2（affects CCT and RGB 2） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 27 | Color Macro 2（override RGB /CCT 2） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 28 | Color Macro Cross Fade 2 (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 29 | Effect（override RGB 1+2/CCT 1+2/Color Macro 1+2） | 000-015，empty 016-039，effect 1（from slow to fast） 040-063，effect 2（from slow to fast） 064-087，effect 3（from slow to fast） 088-111，effect 4（from slow to fast） 112-135，effect 5（from slow to fast） 136-159，effect 6（from slow to fast） 160-183，effect 7（from slow to fast） 184-207，effect 8（from slow to fast） 208-231，effect 9（from slow to fast） 232-255，effect 10（from slow to fast） |
| 30 | Dimmer Curve | 0-51，Current Preset from Menu Settings 52-101，Linear dimmer curve 102-152，Square dimmer curve 153-203，Anti-square dimmer curve 204-255，S-line dimmer curve |
| 31 | Dimmer Response | 0-63，Current Preset from Menu Settings 64-127，LED/fast 128-191，Medium 192-255，Halogen/slow |
| 32 | Redshift | 0-84，Current Preset from Menu Settings 85-170，Off 171-255，On |
| 33 | Device Settings | The push rod needs to be pushed for the first time to cause a change in the value before it can be considered as a setting. The following functions require a 3-second pause to take effect 000-029 No function 030-034 8bit 035-039 16bit 040-044 No function 045-049 DMX Fail Blackout 050-054 DMX Fail Hold 055-059 DMX Fail - Emergency Light 060-064 No function 065-069 Linear dimming curve 070-074 Square dimming curve 075-079 Anti-square dimming curve 080-084 S dimming curve 085-089 No function 090-094 Dimmer Response fast 095-099 Dimmer Response medium 100-104 Dimmer Response slow(halogen) 105-109 No function 110-114 Redshift on（affects only between 2700-3500K） 115-119 Redshift off 120-139 No function 140-144 LED Frequency 1kHz 145-149 LED Frequency 2kHz 150-154 LED Frequency 4kHz 155-159 LED Frequency 12kHz 160-164 LED Frequency 16kHz 165-169 LED Frequency 25kHz 170-174 No function 175-179 Fan Auto 180-184 Fan Silent 185-189 Fan Off 190-194 Fan High Power 195-199 No function 200-214 Invert Mapping Off 215-219 Invert Mapping On 220-224 No function 225-229 LED Mode - low-power 230-234 LED Mode - High-power 235-239 No function 240-255 The current setting is invalid |

10CH DIRECT Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | R1 dimmer |  |
| 2 | G1 dimmer |  |
| 3 | B1 dimmer |  |
| 4 | A1 dimmer |  |
| 5 | W1 dimmer |  |
| 6 | R2 dimmer |  |
| 7 | G2 dimmer |  |
| 8 | B2 dimmer |  |
| 9 | A2 dimmer |  |
| 10 | W2 dimmer |  |

12CH DIRECT Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 3 | R dimmer |  |
| 4 | G dimmer |  |
| 5 | B dimmer |  |
| 6 | A dimmer |  |
| 7 | W dimmer |  |
| 8 | CCT(affects RGB) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 9 | Tint（affects CCT and RGB） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 10 | Color Macro（override RGB/CCT） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 11 | Color Macro Cross fade (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 12 | Device Settings | The push rod needs to be pushed for the first time to cause a change in the value before it can be considered as a setting. The following functions require a 3-second pause to take effect 000-029 No function 030-034 8bit 035-039 16bit 040-044 No function 045-049 DMX Fail Blackout 050-054 DMX Fail Hold 055-059 DMX Fail - Emergency Light 060-064 No function 065-069 Linear dimming curve 070-074 Square dimming curve 075-079 Anti-square dimming curve 080-084 S dimming curve 085-089 No function 090-094 Dimmer Response fast 095-099 Dimmer Response medium 100-104 Dimmer Response slow(halogen) 105-109 No function 110-114 Redshift on（affects only between 2700-3500K） 115-119 Redshift off 120-139 No function 140-144 LED Frequency 1kHz 145-149 LED Frequency 2kHz 150-154 LED Frequency 4kHz 155-159 LED Frequency 12kHz 160-164 LED Frequency 16kHz 165-169 LED Frequency 25kHz 170-174 No function 175-179 Fan Auto 180-184 Fan Silent 185-189 Fan Off 190-194 Fan High Power 195-199 No function 200-214 Invert Mapping Off 215-219 Invert Mapping On 220-224 No function 225-229 LED Mode - low-power 230-234 LED Mode - High-power 235-239 No function 240-255 The current setting is invalid |

17CH DIRECT Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 3 | Dimmer 1 |  |
| 4 | R1 dimmer |  |
| 5 | G1 dimmer |  |
| 6 | B1 dimmer |  |
| 7 | A1 dimmer |  |
| 8 | W1 dimmer |  |
| 9 | Dimmer 2 |  |
| 10 | R2 dimmer |  |
| 11 | G2 dimmer |  |
| 12 | B2 dimmer |  |
| 13 | A2 dimmer |  |
| 14 | W2 dimmer |  |
| 15 | CCT(affects RGBAW 1+2) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 16 | Tint（affects CCT and RGBAW 1+2） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 17 | Device Settings | The push rod needs to be pushed for the first time to cause a change in the value before it can be considered as a setting. The following functions require a 3-second pause to take effect 000-029 No function 030-034 8bit 035-039 16bit 040-044 No function 045-049 DMX Fail Blackout 050-054 DMX Fail Hold 055-059 DMX Fail - Emergency Light 060-064 No function 065-069 Linear dimming curve 070-074 Square dimming curve 075-079 Anti-square dimming curve 080-084 S dimming curve 085-089 No function 090-094 Dimmer Response fast 095-099 Dimmer Response medium 100-104 Dimmer Response slow(halogen) 105-109 No function 110-114 Redshift on（affects only between 2700-3500K） 115-119 Redshift off 120-139 No function 140-144 LED Frequency 1kHz 145-149 LED Frequency 2kHz 150-154 LED Frequency 4kHz 155-159 LED Frequency 12kHz 160-164 LED Frequency 16kHz 165-169 LED Frequency 25kHz 170-174 No function 175-179 Fan Auto 180-184 Fan Silent 185-189 Fan Off 190-194 Fan High Power 195-199 No function 200-214 Invert Mapping Off 215-219 Invert Mapping On 220-224 No function 225-229 LED Mode - low-power 230-234 LED Mode - High-power 235-239 No function 240-255 The current setting is invalid |

28CH DIRECT Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 3 | Duration | Extend the lighting time during flicker, only applicable to the normal flicker of channel 2（025-064） |
| 4 | Dimmer 1 |  |
| 5 | R1 dimmer |  |
| 6 | G1 dimmer |  |
| 7 | B1 dimmer |  |
| 8 | A1 dimmer |  |
| 9 | W1 dimmer |  |
| 10 | CCT 1(affects RGBAW 1) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 11 | Tint 1（affects CCT and RGBAW 1） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 12 | Color Macro 1（override RGBAW /CCT 1） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 13 | Color Macro Cross Fade 1 (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 14 | Dimmer 2 |  |
| 15 | R2 dimmer |  |
| 16 | G2 dimmer |  |
| 17 | B2 dimmer |  |
| 18 | A2 dimmer |  |
| 19 | W2 dimmer |  |
| 20 | CCT 2(affects RGBAW 1) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 21 | Tint 2（affects CCT and RGBAW 2） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 22 | Color Macro 2（override RGBAW /CCT 2） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 23 | Color Macro Cross Fade 2 (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 24 | Effect（override RGB 1+2/CCT 1+2/Color Macro 1+2） | 000-015，empty 016-039，effect 1（from slow to fast） 040-063，effect 2（from slow to fast） 064-087，effect 3（from slow to fast） 088-111，effect 4（from slow to fast） 112-135，effect 5（from slow to fast） 136-159，effect 6（from slow to fast） 160-183，effect 7（from slow to fast） 184-207，effect 8（from slow to fast） 208-231，effect 9（from slow to fast） 232-255，effect 10（from slow to fast） |
| 25 | Dimmer Curve | 0-51，Current Preset from Menu Settings 52-101，Linear dimmer curve 102-152，Square dimmer curve 153-203，Anti-square dimmer curve 204-255，S-line dimmer curve |
| 26 | Dimmer Response | 0-63，Current Preset from Menu Settings 64-127，LED/fast 128-191，Medium 192-255，Halogen/slow |
| 27 | Redshift | 0-84，Current Preset from Menu Settings 85-170，Off 171-255，On |
| 28 | Device Settings | The push rod needs to be pushed for the first time to cause a change in the value before it can be considered as a setting. The following functions require a 3-second pause to take effect 000-029 No function 030-034 8bit 035-039 16bit 040-044 No function 045-049 DMX Fail Blackout 050-054 DMX Fail Hold 055-059 DMX Fail - Emergency Light 060-064 No function 065-069 Linear dimming curve 070-074 Square dimming curve 075-079 Anti-square dimming curve 080-084 S dimming curve 085-089 No function 090-094 Dimmer Response fast 095-099 Dimmer Response medium 100-104 Dimmer Response slow(halogen) 105-109 No function 110-114 Redshift on（affects only between 2700-3500K） 115-119 Redshift off 120-139 No function 140-144 LED Frequency 1kHz 145-149 LED Frequency 2kHz 150-154 LED Frequency 4kHz 155-159 LED Frequency 12kHz 160-164 LED Frequency 16kHz 165-169 LED Frequency 25kHz 170-174 No function 175-179 Fan Auto 180-184 Fan Silent 185-189 Fan Off 190-194 Fan High Power 195-199 No function 200-214 Invert Mapping Off 215-219 Invert Mapping On 220-224 No function 225-229 LED Mode - low-power 230-234 LED Mode - High-power 235-239 No function 240-255 The current setting is invalid |

41CH DIRECT Channel Description

| CH | Function | Description |
| --- | --- | --- |
| 1 | Dimmer |  |
| 2 | Dimmer fine-tuning |  |
| 3 | Strobe | If the relevant flashing is not executed, the value needs to be adjusted to the "Shutter open" area at least 0-19，Shutter close 20-24，Shutter open 25-64，Normal strobe（fast-slow） 65-69，Shutter open 70-109，Opening pulse（fast-slow） 110-114，Shutter open 115-154，Closing pulse（fast-slow） 155-159，Shutter open 160-199，Random strobe（fast-slow） 200-204，Shutter open 205-244，Lightning strobe（fast-slow） 245-255，Shutter open |
| 4 | Duration | Extend the lighting time during flicker, only applicable to the normal flicker of channel 2（025-064） |
| 5 | Dimmer 1 |  |
| 6 | Dimmer 1 fine-tuning |  |
| 7 | R1 dimmer |  |
| 8 | R1 dimmer fine-tuning |  |
| 9 | G1 dimmer |  |
| 10 | G1 dimmer fine-tuning |  |
| 11 | B1 dimmer |  |
| 12 | B1 dimmer fine-tuning |  |
| 13 | A1 dimmer |  |
| 14 | A1 dimmer fine-tuning |  |
| 15 | W1 dimmer |  |
| 16 | W1 dimmer fine-tuning |  |
| 17 | CCT 1(affects RGBAW 1) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 18 | Tint 1（affects CCT and RGBAW 1） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 19 | Color Macro 1（override RGBAW /CCT 1） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 20 | Color Macro Cross Fade 1 (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 21 | Dimmer 2 |  |
| 22 | Dimmer 2 fine-tuning |  |
| 23 | R2 dimmer |  |
| 24 | R2 dimmer fine-tuning |  |
| 25 | G2 dimmer |  |
| 26 | G2 dimmer fine-tuning |  |
| 27 | B2 dimmer |  |
| 28 | B2 dimmer fine-tuning |  |
| 29 | A2 dimmer |  |
| 30 | A2 dimmer fine-tuning |  |
| 31 | W2 dimmer |  |
| 32 | W2 dimmer fine-tuning |  |
| 33 | CCT 2(affects RGBAW 1) | 000-004,5600K 005-255, 2000K-10000K For every 3-4 increase in push rod value, the color temperature increases by a step of 100K |
| 34 | Tint 2（affects CCT and RGBAW 2） | 001-127 ,Megenta->Neutral 0,128 ,Neutral 129-255, Neutral->Green |
| 35 | Color Macro 2（override RGBAW /CCT 2） | 000,empty 001-255，Uniformly distribute 60 colored papers, please refer to the color paper table below for details |
| 36 | Color Macro Cross Fade 2 (Transition Time between Color Macros) | 000-005,empty 006-255，The transition time is getting longer and longer |
| 37 | Effect（override RGB 1+2/CCT 1+2/Color Macro 1+2） | 000-015，empty 016-039，effect 1（from slow to fast） 040-063，effect 2（from slow to fast） 064-087，effect 3（from slow to fast） 088-111，effect 4（from slow to fast） 112-135，effect 5（from slow to fast） 136-159，effect 6（from slow to fast） 160-183，effect 7（from slow to fast） 184-207，effect 8（from slow to fast） 208-231，effect 9（from slow to fast） 232-255，effect 10（from slow to fast） |
| 38 | Dimmer Curve | 0-51，Current Preset from Menu Settings 52-101，Linear dimmer curve 102-152，Square dimmer curve 153-203，Anti-square dimmer curve 204-255，S-line dimmer curve |
| 39 | Dimmer Response | 0-63，Current Preset from Menu Settings 64-127，LED/fast 128-191，Medium 192-255，Halogen/slow |
| 40 | Redshift | 0-84，Current Preset from Menu Settings 85-170，Off 171-255，On |
| 41 | Device Settings | The push rod needs to be pushed for the first time to cause a change in the value before it can be considered as a setting. The following functions require a 3-second pause to take effect 000-029 No function 030-034 8bit 035-039 16bit 040-044 No function 045-049 DMX Fail Blackout 050-054 DMX Fail Hold 055-059 DMX Fail - Emergency Light 060-064 No function 065-069 Linear dimming curve 070-074 Square dimming curve 075-079 Anti-square dimming curve 080-084 S dimming curve 085-089 No function 090-094 Dimmer Response fast 095-099 Dimmer Response medium 100-104 Dimmer Response slow(halogen) 105-109 No function 110-114 Redshift on（affects only between 2700-3500K） 115-119 Redshift off 120-139 No function 140-144 LED Frequency 1kHz 145-149 LED Frequency 2kHz 150-154 LED Frequency 4kHz 155-159 LED Frequency 12kHz 160-164 LED Frequency 16kHz 165-169 LED Frequency 25kHz 170-174 No function 175-179 Fan Auto 180-184 Fan Silent 185-189 Fan Off 190-194 Fan High Power 195-199 No function 200-214 Invert Mapping Off 215-219 Invert Mapping On 220-224 No function 225-229 LED Mode - low-power 230-234 LED Mode - High-power 235-239 No function 240-255 The current setting is invalid |

COLOR CARD

| VALUE | COLOR | VALUE | COLOR |
| --- | --- | --- | --- |
| 1 | Oyster white | 31 | Shi Nanzi |
| 2 | Beige | 32 | Dingxiang Blue |
| 3 | golden | 33 | Pearl Purple |
| 4 | Beige color | 34 | Pearl Blackberry |
| 5 | Ivory color | 35 | Pigeon Blue |
| 6 | Light ivory color | 36 | Sky blue |
| 7 | dark yellow | 37 | Light Blue |
| 8 | canary yellow | 38 | Gentian Blue |
| 9 | Light Orange Yellow | 39 | Bright Blue |
| 10 | Yellow Orange | 40 | Changing Blue |
| 11 | Light Orange | 41 | Water Blue |
| 12 | Pure Orange | 42 | Far Blue |
| 13 | Salmon Orange | 43 | wathet |
| 14 | Pearl Orange | 44 | Pearl Gentian Blue |
| 15 | Flame Red | 45 | Pearl Night Blue |
| 16 | Signal red | 46 | patina green |
| 17 | Carmine | 47 | Emerald Green |
| 18 | Ruby Red | 48 | Leaf green |
| 19 | Purple red | 49 | Grass green |
| 20 | Tomato red | 50 | Yellow green |
| 21 | Antique pink | 51 | Green traffic |
| 22 | rose pink | 52 | Fern green |
| 23 | Coral Red | 53 | Opal green |
| 24 | rose | 54 | Light green |
| 25 | Traffic Red | 55 | Pastel turquoise |
| 26 | Purple red | 56 | Silk Grey |
| 27 | Eastern Red | 57 | Cream color |
| 28 | Wine Red | 58 | Signal white |
| 29 | Oxidation Red | 59 | Light brown |
| 30 | Red lilac | 60 | Orange yellow brown |