# 60x3w RGBW LED Par Light

LED display operation center

after power on,press MENU to view Main menu,press

UP/DOWN to choose sub-menu,then press ENTER to save.

d001

d512

A001

A512

MENU  UP    DOWN   ENTER

4CH mode

8CH mode

CL27

RGB color select

CL01

FF01

FF99

RGB color gradual change

CC01

CC99

RGB color jump

r255

G255

b255

w255

Red manual dimmer

r001. Green manual dimmer

Blue manual dimmer

FF01

White manual dimmer

dEoo

dEoo

Demo

SoUd

SoUd

Sound control

SLAv

Slave mode

SLAv

DMX channels

| 4CH mode |  |  |
| --- | --- | --- |
| Channel | Value | Function |
| CH1 | 0 | Red off |
|  | 1-255 | 0-100% dimmer |
| CH2 | 0 | Green off |
|  | 1-255 | 0-100% dimmer |
| CH3 | 0 | Blue off |
|  | 1-255 | 0-100% dimmer |
| CH4 | 0 | White off |
|  | 1-255 | 0-100% dimmer |
| 8CH mode |  |  |
| Channel | Value | Function |
| CH1 | 0-255 | General dimmer |
| CH2 | 0 | Red off |
|  | 1-255 | 0-100% dimmer |
| CH3 | 0 | Green off |
|  | 1-255 | 0-100% dimmer |
| CH4 | 0 | Blue off |
|  | 1-255 | 0-100% dimmer |
| CH5 | 0 | White off |
|  | 1-255 | 0-100% dimmer |
| CH6 | 0-255 | Strobe slow to fast |
| CH7 | 0-255 | Speed slow to fast |
| CH8 | 0-50 | Open CH1-CH6 |
|  | 51-100 | Programme colors |
|  | 101-150 | Color gradual change |
|  | 151-200 | Color fade |
|  | 201-250 | Color jump |
|  | 251-255 | Open Sound mode |

Set Master-Slave

2 lights or more connected with DMX cable,one light set as Demo or color auto change,the others all set as Slave mode.