COLOR LIGHT PRO

User Manual

The W7 User Manual includes a description. safety precautions. installation, programming. operation and maintenance instructions for the W7 as of the release date of this edition.

The W7 User Manual includes a description. safety precautions, installation, programming. operation and maintenance instructions for the C7 as of the release date of this edition.

Edition Notes

W7 User Manual includes a description, safety precautions, installation, programming,operation and maintenance instructions for W7 as of the release date of this edition.

Copyright Notice

The works of authorship contained in this manual, including, but not limited to, all designs, text, and images

are owned by OUR COMPANY.

Manual Use

OUR COMPANY authorizes its customers to download and print this manual for professional information purposes only. OUR COMPANY expressly prohibits the usage, copy, storage, distribution, modification, or printing of this manual or its content for any other purpose without written consent from OUR COMPANY.

Document Printing

For best results, print this document in color, on letter size paper (8.5 x 11 in), double-sided. If using A4

paper (210 x 297 mm), configure the printer to scale the content accordingly.

Intended Audience

Any person installing, operating, and/or maintaining this product should completely read through the guide

that shipped with the product, as well as this manual, before installing, operating, or maintaining this product.

Disclaimer

OUR COMPANY believes that the information contained in this manual is accurate in all respects. However,

OUR COMPANY assumes no responsibility and specifically disclaims any and all liability to any party for any loss,damage or disruption caused by any errors or omissions in this document, whether such errors or

omissions result from negligence, accident or any other cause.

OUR COMPANY reserves the right to revise the content of this document without any obligation to notify any person or company of such revision

TABLE OF CONTENTS

1.Before You Begin......................................................................................................................... 3

What Is Included............................................................................................................................. 3

Text Conventions............................................................................................................................ 3 Symbols.......................................................................................................................................... 3

Safety Notes................................................................................................................................... 3

2. Introduction................................................................................................................................. 5

Description...................................................................................................................................... 5 Features.......................................................................................................................................... 5

Product Overview............................................................................................................................ 5

Product Dimensions........................................................................................................................ 6

3. Setup........................................................................................................................................... 6

AC Power........................................................................................................................................ 6

AC Plug........................................................................................................................................... 6

DMX Linking.................................................................................................................................... 6

DMX Personalities........................................................................................................................... 6

Remote Device Management.......................................................................................................... 6

Mounting......................................................................................................................................... 8

Orientation...................................................................................................................................... 8

Rigging........................................................................................................................................... 8

Product Mounting Diagram............................................................................................................. 8

4. Operation.................................................................................................................................... 9

Control Panel Description............................................................................................................... 9

Programming.................................................................................................................................. 9

Control Personalities...................................................................................................................... 9

Starting Address............................................................................................................................. 9

Menu Map...................................................................................................................................... 10

DMX Values................................................................................................................................... 12

10CH……...................................................................................................................................... 12 55CH............................................................................................................................................. 17

64CH............................................................................................................................................. 19

06CH............................................................................................................................................. 24

21CH............................................................................................................................................. 26

Plate Patterns............................................................................................................................... 34

Beam Patterns.............................................................................................................................. 35

Pixel Mapping............................................................................................................................... 36

5. Maintenance............................................................................................................................. 36

1. Before You Begin

What Is Included：

• COLOR  LIGHT PRO W7  *1

• Seetronic Powerkon IP65 power cable  *1

• Seetronic Powerkon IP65 signal cable（Optional）

• Safety rope  *1（Optional）

• Omega bracket with mounting hardware  *2  or  Folded clamps  *1（Optional）

• Cartoon box  *1  or  Flight case<4 in 1>  *1（Optional）

Text Conventions

| Convention | Meaning |
| --- | --- |
| 1-512 | A range of values |
| 50/60 | A set of values of which only one can be chosen |
| Settings | A menu option not to be modified |
| <ENTER> | A key to be pressed on the product’s control panel |

Symbols

| Symbol | Meaning |
| --- | --- |
|  | Critical installation, configuration, or operation information. Not following these instructions may make the product not work, cause damage to the product, or cause harm to the operator. |
|  | Important installation or configuration information. The product may not function correctly if this information is not used. |
|  | Useful information. |

|  | Any reference to data or power connections in this manual assumes the use of Seetronic IP-rated cables. |
| --- | --- |
|  | The term “DMX” used throughout this manual refers to the USITT DMX512-A digital data transmission protocol. Connection of the control signal: DMX line •The product has XLR sockets for DMX input and output. •Notice: This control circuit is isolated and belongs to the Class 2 data port. The control circuit has a cumulative leakage current of less than 3.5 mA. |

Safety Notes

Read all the following safety notes before working with this product. These notes contain important information about the installation, usage, and maintenance of this product.

|  | This product contains no user-serviceable parts. Any reference to servicing in this User Manual will only apply to properly trained, certified technicians. Do not open the housing or attempt any repairs. All applicable local codes and regulations apply to proper installation of this product. |
| --- | --- |

•The luminaire is intended for professional use only.

•If the external flexible cable or cord of this luminaire is damaged, it shall be replaced by a special cord or cord exclusively available from the manufacturer or its service agent.

•The light source contained in this luminaire shall only be replaced by the manufacturer or its service agent or a similar qualified person.

•CAUTION:

•This product’s housing may be hot when operating. Mount this product in a location with adequate ventilation, at least 20 in (50 cm) from adjacent surfaces.

•When transferring the product from extreme temperature environments, (e.g., cold truck to warm humid ballroom) condensation may form on the internal electronics of the product. To avoid causing a failure, allow the product to fully acclimate to the surrounding environment before connecting it to power.

•Flashing light is known to trigger epileptic seizures. User must comply with local laws regarding notification of strobe use.

•ALWAYS:

•Disconnect from power before cleaning the product or replacing the fuse.

•When using an IP65-rated product in an outdoor environment, use IP65- (or higher) rated power and data cable.

•Replace and secure IP-rated protective covers to all power, data, USB, or other ports when not in use.

•Replace the fuse with the same type and rating.

•Use a safety cable when mounting this product overhead.

•Connect this product to a grounded and protected circuit.

•DO NOT:

•Open this product. It contains no user-serviceable parts.

•Look at the light source when the product is on.

•Leave any flammable material within 20 cm of this product while operating or connected to power.

•Connect this product to a dimmer or rheostat.

•Operate this product if the housing, lenses, or cables appear damaged.

•Submerge this product (adhere to standards for the published IP rating). Regular outdoor operation is fine.

•Permanently install outdoors in locations with extreme environmental conditions. This includes, but is not

limited to:

•Exposure to a marine/saline environment (within 3 miles of a saltwater body of water).

•Locations where normal temperatures exceed the temperature ranges in this manual.

•Locations that are prone to flooding or being buried in snow.

•Other areas where the product will be subject to extreme radiation or caustic substances.

•ONLY use the handles or the hanging/mounting brackets to carry this product.

•The maximum ambient temperature is 113 °F (45 °C). Do not operate this product at higher temperatures.

•The minimum startup temperature is -4°F (-20°C). Do not start the product at lower temperatures.

•The minimum ambient temperature is 14°F (-10°C). Do not operate the product at lower temperatures.

•To eliminate unnecessary wear and improve its lifespan, during periods of non-use completely disconnect the product from power via breaker or by unplugging it.

•In the event of a serious operating problem, stop using immediately.

|  | If a C7 product requires service, contact C7 Technical Support. |
| --- | --- |

2. Introduction

Description

C7 is an IP65-rated motorized strobe/wash with two ultra-bright, white light tube elements surrounded by an electrifying, color-mixing, and pixel-mappable face. This versatile strobe light can double as a wash fixture with 180° tilt range to throw bright saturated color wherever needed. The IP65 casing protects the fixture from any environmental factors or weather and has a folded clamps to mount the fixture horizontally or vertically without any hassle. Multiple programming personalities give a varying amount of control, from simple setups with a multitude of built-in effects, or wider control options to take advantage of the pixel-mappable zones: 14 in the RGB face and 28 in the two tube elements. Built-in alignment pins insure perfectly straight linear arrays to create massive arrangements. DMX, Art-Net, and sACN control. All LEDs feature 16-bit dimming for smooth fades and Pulse Width Modulation (PWM) for on-camera use.

Features

•High-power, 180° tilting hybrid strobe/wash with IP65 rating for all-weather use

•14 controllable sections of RGB LEDs and 28 sections of control of the white strobe LEDs

•Multiple mounting locations and a yoke that can be repositioned with ¼ turn adapters for creative and convenient rigging options in vertical and horizontal orientations

•Multitude of built-in effect macros with control parameters

•DMX, Art-Net, and sACN control capability

•16-bit dimming control for smooth, perfect fades

•Easy-to-use OLED display

•Rugged design for years of rough use in the most extreme conditions

•Adjustable PWM and Remote Device Management (RDM) for added flexibility

•USB port for fixture software updates

•Built-in alignment pins to ensure perfectly straight linear arrays

Product Overview

| # | name |
| --- | --- |
| 1 | Power in |
| 2 | 3 or 5-pin DMX in/out |
| 3 | Network in/out |
| 4 | Touchscreen display/control panel |
| 5 | Folded clamps |

3. Setup

AC Power

The C7 has an auto-ranging power supply, and it can work with an input voltage range of 100 to 240 VAC, 50/60Hz.

To determine the product’s power requirements (circuit breaker, power outlet, and wiring), use the current value listed on the label affixed to the product’s back panel, or refer to the product’s specifications chart. The listed current rating indicates the product’s average current draw under normal conditions.

|  | • Always connect the product to a protected circuit (a circuit breaker or fuse). Make sure the product has an appropriate electrical ground to avoid the risk of electrocution or fire. • To eliminate unnecessary wear and improve its lifespan, during periods of non-use completely disconnect the product from power via breaker or by unplugging it. |
| --- | --- |
|  | All applicable local codes and regulations apply to proper installation of this product. |

AC Plug

The C7 comes with a power input cable terminated with a Seetronic Powerkon IP65 connector on one end and an Edison plug on the other end (U.S. market).

DMX Linking

The W7 can be linked to a DMX controller using a 3-pin DMX connection. If using other DMX compatible products with this product, it is possible to control each individually with a single DMX controller.

DMX Personalities

•Refer to the Introduction for a brief description of each DMX personality.

•Refer to the Operation chapter to learn how to configure theW7 to work in these personalities.

•The Control Channel Assignments and Values section provides detailed information regarding the DMX personalities.

Remote Device Management

Remote Device Management (RDM) is a standard for allowing DMX-enabled devices to communicate bi- directionally along existing DMX cabling. Check the DMX controller’s User Manual or with the manufacturer as not all DMX controllers have this capability. The W7 supports RDM protocol that allows feedback to make changes to menu map options.

Mounting

Before mounting the product, read and follow the safety recommendations indicated in the Safety Notes.

Orientation

Always mount this product in a safe position, making sure there is adequate room for ventilation, configuration, and maintenance.

Rigging

OUR COMPANY recommends using the following general guidelines when mounting this product.

• Before deciding on a location for the product, always make sure there is easy access to the product for maintenance and programming.

• Make sure that the structure and attachment points can support the weight before hanging the product (see the Technical Specifications for weight information).

• When mounting the product overhead, always use a safety cable. Mount the product securely to a rigging point, whether an elevated platform or a truss.

• When rigging the product onto a truss, use a mounting clamp of appropriate weight capacity.

Safety cable

4. Operation

Control Panel Description

| Button | Function |
| --- | --- |
| <MENU> | Exits from the current menu or function |
| <UP> | Enables the selected menu or sets the selected value in to the current function |
| <DOWN> | Navigates upward through the menu list or increases the numeric value when in a function |
| <ENTER> | Navigates downward through the menu list or decreases the numeric value when in a function |

Programming

Refer to the Menu Map to understand the menu options. The menu map shows the main level and a variable number of programming levels for each option.

• To go to the desired main level, press <MENU>repeatedly until the option shows on the display. Press <ENTER>

to select. This will show the first programming level for that option.

• To select an option or value within the current programming level, press <UP> OR <DOWN>until the option shows on the display. Press <ENTER> to select. This will show the selected value or the first option of the next programming level.

• press <MENU>repeatedly to exit to the previous main level.

Control Personalities

To set the control personality:

1. Go to the DMX Channel main level.

2. Select the desired personality, from CH10/CH55/CH64/CH06/CH21

|  | • See the Starting Address personality. section for the highest starting address available for each personality. • Make sure that the starting addresses on the various products do not overlap due to the new personality setting. |
| --- | --- |

Starting Address

Each product will respond to a unique starting address from the controller. All products with the same starting address will respond in unison. To set the starting address:

1. Go to the Start Address main level.

2. Select the starting address (1–512).

• The highest recommended starting address for 14CH is 499.

• The highest recommended starting address for 18CH is 495.

• The highest recommended starting address for 30CH is 483.

• The highest recommended starting address for 66CH is 447.

• The highest recommended starting address for 70CH is 443.

Menu Map

| Main Menu | Submenu | Menu function options and instructions |  |
| --- | --- | --- | --- |
| DMX SETTINGS | DMX Address | Set the DMX start address |  |
|  |  | 1-499 (06CH) |  |
|  |  | 1-495 (10CH) |  |
|  |  | 1-483 (55CH) |  |
|  |  | 1-447 (64CH) |  |
|  |  | 1-443 (21CH) |  |
|  |  | / |  |
|  |  | / |  |
|  | DMX Channel Mode | Set the DMX channel mode |  |
|  |  | 06CH |  |
|  |  | 10CH |  |
|  |  | 55CH |  |
|  |  | 64CH |  |
|  |  | 21CH |  |
|  |  | / |  |
|  | No DMX Status | Blackout | Takes all traits to 0 when DMX signal is lost or interrupted. |
|  |  | Hold | Holds all traits at last received value when DMX signal is lost or interrupted. |
|  |  | Manual | Defaults to values set in Manual Mode when DMX signal is lost or interrupted. |
|  | Connect | DMX |  |
|  | Network | IP address XXX.XXX.XXX.XXX |  |
|  |  | Subnet Mask XXX.XXX.XXX.XXX |  |
|  | Art-Net Settings | Net (0-127) |  |
|  | sACN Settings |  |  |

DMX Chrat

| 10通道模式 | 功能 | DMX值 | 描述 |
| --- | --- | --- | --- |
| 1 | Dimmer | 0-255 | 0-100% |
| 2 | Strobe | 0-19 | No Effect |
|  |  | 20-255 | Strobe slow to fast |
| 3 | Red | 0-255 | 0-100% |
| 4 | Green | 0-255 | 0-100% |
| 5 | Blue | 0-255 | 0-100% |
| 6 | White | 0-255 | 0-100% |
| 7 | Color Macro | 0-10 | No Function |
|  |  | 11-30 | R 100%,G 50-100-50%,B 0 |
|  |  | 31-50 | R 100%-0,G 100%, B,0 |
|  |  | 51-70 | R 0,G 100%,B 0-100% |
|  |  | 71-90 | R 0,G 100%-0,B 100% |
|  |  | 91-110 | R 0-100%, G 0, B 100% |
|  |  | 111-130 | R 100%,G 0, B 100%-0 |
|  |  | 131-150 | R 100% ,G 0-100% ,B 0-100% |
|  |  | 151-170 | R 100%-0 , G 100%-0 , B 100% |
|  |  | 171-200 | R 100%,G 100%, B 100% |
|  |  | 201-205 | Color 1 |
|  |  | 206-210 | Color 2 |
|  |  | 211-215 | Color 3 |
|  |  | 216-220 | Color 4 |
|  |  | 221-225 | Color 5 |
|  |  | 226-230 | Color 6 |
|  |  | 231-235 | Color 7 |
|  |  | 236-240 | Color 8 |
|  |  | 241-245 | Color 9 |
|  |  | 246-250 | Color 10 |
|  |  | 251-255 | Color 11 |
| 8 | Background | 0-10 | No Function |
|  |  | 11-255 | Background Color |
| 9 | Background Dimmer | 0-255 | 0-100% |
| 9 | Show | 0-10 | No Function |
|  |  | 11-15 | Show1 |
|  |  | 16-20 | Show2 |
|  |  | 21-25 | Show3 |
|  |  | 26-30 | Show4 |
|  |  | 31-35 | Show5 |
|  |  | 36-40 | Show6 |
|  |  | 41-45 | Show7 |
|  |  | 46-50 | Show8 |
|  |  | 51-55 | Show9 |
|  |  | 56-60 | Show10 |
|  |  | 61-65 | Show11 |
|  |  | 66-70 | Show12 |
|  |  | 71-75 | Show13 |
|  |  | 76-80 | Show14 |
|  |  | 81-85 | Show15 |
|  |  | 86-90 | Show16 |
|  |  | 91-95 | Show17 |
|  |  | 96-100 | Show18 |
|  |  | 101-105 | Show19 |
|  |  | 106-110 | Show20 |
|  |  | 111-115 | Show21 |
|  |  | 116-120 | Show22 |
|  |  | 121-125 | Show23 |
|  |  | 126-130 | Show24 |
|  |  | 131-135 | Show25 |
|  |  | 136-140 | Show26 |
|  |  | 141-145 | Show27 |
|  |  | 146-150 | Show28 |
|  |  | 151-155 | Show29 |
|  |  | 156-160 | Show30 |
|  |  | 161-165 | Show31 |
|  |  | 166-170 | Show32 |
|  |  | 171-175 | Show33 |
|  |  | 176-180 | Show34 |
|  |  | 181-185 | Show35 |
|  |  | 186-190 | Show36 |
|  |  | 191-195 | Show37 |
|  |  | 196-200 | Show38 |
|  |  | 201-205 | Show39 |
|  |  | 206-210 | Show40 |
|  |  | 211-215 | Show41 |
|  |  | 216-220 | Show42 |
|  |  | 221-225 | Show43 |
|  |  | 226-230 | Show44 |
|  |  | 231-235 | Show45 |
|  |  | 236-245 | Sound1 |
|  |  | 246-255 | Sound2 |
| 10 | Show Speed | 0-255 | Show Speed Slow to Fast |
| 55通道模式 | 功能 | DMX值 | 描述 |
| 1 | Dimmer | 0-255 | 0-100% |
| 2 | Strobe | 0-19 | No Effect |
|  |  | 20-255 | Strobe slow to fast |
| 3 | Red1 | 0-255 | 0-100% |
| 4 | Green1 | 0-255 | 0-100% |
| 5 | Blue1 | 0-255 | 0-100% |
| 6 | White1 | 0-255 | 0-100% |
|  | … |  |  |
| 47 | Red16 | 0-255 | 0-100% |
| 48 | Green16 | 0-255 | 0-100% |
| 49 | Blue16 | 0-255 | 0-100% |
| 50 | White16 | 0-255 | 0-100% |
| 51 | Color Macro | 0-10 | No Function |
|  |  | 11-30 | R 100%,G 50-100-50%,B 0 |
|  |  | 31-50 | R 100%-0,G 100%, B,0 |
|  |  | 51-70 | R 0,G 100%,B 0-100% |
|  |  | 71-90 | R 0,G 100%-0,B 100% |
|  |  | 91-110 | R 0-100%, G 0, B 100% |
|  |  | 111-130 | R 100%,G 0, B 100%-0 |
|  |  | 131-150 | R 100% ,G 0-100% ,B 0-100% |
|  |  | 151-170 | R 100%-0 , G 100%-0 , B 100% |
|  |  | 171-200 | R 100%,G 100%, B 100% |
|  |  | 201-205 | Color 1 |
|  |  | 206-210 | Color 2 |
|  |  | 211-215 | Color 3 |
|  |  | 216-220 | Color 4 |
|  |  | 221-225 | Color 5 |
|  |  | 226-230 | Color 6 |
|  |  | 231-235 | Color 7 |
|  |  | 236-240 | Color 8 |
|  |  | 241-245 | Color 9 |
|  |  | 246-250 | Color 10 |
|  |  | 251-255 | Color 11 |
| 52 | Background | 0-10 | No Function |
|  |  | 11-255 | Background Color |
| 53 | Background Dimmer | 0-255 | 0-100% |
| 54 | Show | 0-10 | No Function |
|  |  | 11-15 | Show1 |
|  |  | 16-20 | Show2 |
|  |  | 21-25 | Show3 |
|  |  | 26-30 | Show4 |
|  |  | 31-35 | Show5 |
|  |  | 36-40 | Show6 |
|  |  | 41-45 | Show7 |
|  |  | 46-50 | Show8 |
|  |  | 51-55 | Show9 |
|  |  | 56-60 | Show10 |
|  |  | 61-65 | Show11 |
|  |  | 66-70 | Show12 |
|  |  | 71-75 | Show13 |
|  |  | 76-80 | Show14 |
|  |  | 81-85 | Show15 |
|  |  | 86-90 | Show16 |
|  |  | 91-95 | Show17 |
|  |  | 96-100 | Show18 |
|  |  | 101-105 | Show19 |
|  |  | 106-110 | Show20 |
|  |  | 111-115 | Show21 |
|  |  | 116-120 | Show22 |
|  |  | 121-125 | Show23 |
|  |  | 126-130 | Show24 |
|  |  | 131-135 | Show25 |
|  |  | 136-140 | Show26 |
|  |  | 141-145 | Show27 |
|  |  | 146-150 | Show28 |
|  |  | 151-155 | Show29 |
|  |  | 156-160 | Show30 |
|  |  | 161-165 | Show31 |
|  |  | 166-170 | Show32 |
|  |  | 171-175 | Show33 |
|  |  | 176-180 | Show34 |
|  |  | 181-185 | Show35 |
|  |  | 186-190 | Show36 |
|  |  | 191-195 | Show37 |
|  |  | 196-200 | Show38 |
|  |  | 201-205 | Show39 |
|  |  | 206-210 | Show40 |
|  |  | 211-215 | Show41 |
|  |  | 216-220 | Show42 |
|  |  | 221-225 | Show43 |
|  |  | 226-230 | Show44 |
|  |  | 231-235 | Show45 |
|  |  | 236-245 | Sound1 |
|  |  | 246-255 | Sound2 |
| 55 | Show Speed | 0-255 | Show Speed Slow to Fast |
| 64通道模式 | 功能 | DMX值 | 描述 |
| 1 | Red1 | 0-255 | 0-100% |
| 2 | Green1 | 0-255 | 0-100% |
| 3 | Blue1 | 0-255 | 0-100% |
| 4 | White1 | 0-255 | 0-100% |
|  | … |  |  |
| 61 | Red16 | 0-255 | 0-100% |
| 62 | Green16 | 0-255 | 0-100% |
| 63 | Blue16 | 0-255 | 0-100% |
| 64 | White16 | 0-255 | 0-100% |
| 6通道模式 | 功能 | DMX值 | 描述 |
| 1 | Dimmer | 0-255 | 0-100% |
| 2 | Strobe | 0-19 | No Effect |
|  |  | 20-255 | Strobe slow to fast |
| 3 | Red | 0-255 | 0-100% |
| 4 | Green | 0-255 | 0-100% |
| 5 | Blue | 0-255 | 0-100% |
| 6 | White | 0-255 | 0-100% |
| 21通道模式 | 功能 | DMX值 | 描述 |
| 1 | Dimmer | 0-255 | 0-100% |
| 2 | Strobe | 0-19 | No Effect |
|  |  | 20-255 | Strobe slow to fast |
| 3 | Red | 0-255 | 0-100% |
| 4 | Green | 0-255 | 0-100% |
| 5 | Blue | 0-255 | 0-100% |
| 6 | White | 0-255 | 0-100% |
| 7 | LED1 | 0-255 | 0-100% |
| 8 | LED2 | 0-255 | 0-100% |
|  | … |  |  |
| 20 | LED15 | 0-255 | 0-100% |
| 21 | LED16 | 0-255 | 0-100% |

Pixel Mapping

5. Maintenance

Product Maintenance

Dust build-up reduces light output performance and can cause overheating. This can lead to reduction of the light source’s life and/or mechanical wear. To maintain optimum performance and minimize wear, clean all lighting products at least twice a month. However, be aware that usage and environmental conditions could be contributing factors to increase the cleaning frequency.

To clean the product, follow the instructions below:

1. Unplug the product from power.

2. Wait until the product is at room temperature.

3. Use a vacuum (or dry compressed air) and a soft brush to remove dust collected on the external

surface/vents.

4. Clean all transparent surfaces with a mild soap solution, ammonia-free glass cleaner, or isopropyl alcohol.

5. Apply the solution directly to a soft, lint free cotton cloth or a lens cleaning tissue.

6. Softly drag any dirt or grime to the outside of the transparent surface.

7. Gently polish the transparent surfaces until they are free of haze and lint.

|  | Always dry the transparent surfaces carefully after cleaning them. |
| --- | --- |